appendix a:  realtime physics MECHANICS EXPERIMENT CONFIGURATION FILES

Listed below are the settings in the Experiment Configuration Files used in these labs.  These files are available from Vernier Software for the Logger Pro software and from PASCO Scientific for Data Studio. They are listed here so that the user can set up files for any other compatible hardware and software package.

| Experiment File | Description | Data Collection | Data Handling | Analysis | Display |
| --- | --- | --- | --- | --- | --- |
| Distance (L01A1-1a) | Graphs distance vs. time | Motion sensor 20 points/sec | NA | NA | One set of graph axes with line. Position: 0-4 m Time 0-10 sec |
| Away and Back (L01A1-1) | Graphs position vs. time. | Motion sensor 20 points/sec | NA | NA | One set of graph axes with line. Position: 0-2 m Time 0-15 sec |
| Position Match (L01A1-2) | Graphs position vs. time with preset persistent position-time graph to match. | Motion sensor 20 points/sec | Position Match graph is persistently displayed on the screen for comparison with collected data. | NA | One set of graph axes with line. Position: 0-4 m Time 0-20 sec Persistent graph as in Lab 1 text. |
| Velocity Graphs (L01A2-1) | Graphs velocity vs. time. | Motion sensor 20 points/sec | NA | NA | One set of graph axes with line. Velocity: -1 to +1 m/s Time 0-5 sec |
| Velocity Match (L01A2-1) | Graphs velocity vs. time with preset persistent velocity-time graph to match. | Motion sensor 20 points/sec | Velocity Match graph is persistently displayed on the screen for comparison with collected data. | NA | One set of graph axes with line. Velocity: -1 to +1 m/s Time 0-20 sec Persistent graph as in Lab 1 text. |
| Velocity from Position (L01A3-1) | Graphs position and velocity vs. time. | Motion sensor 20 points/sec | NA | Use analysis feature, statistics feature and graphical fit routine to find average velocity from position-time and velocity-time graphs. | Two sets of graph axes with lines. Position 0-4 m Velocity: -2 to +2 m/s Time 0-5 sec |

| Constant Velocity (L01A4-1) | Graphs position and velocity vs. time. | Motion sensor 20 points/sec | NA | NA | Two sets of graph axes with lines. Position: 0-2 m Velocity: -1 to +1 m/s Time 0-5 sec |
| --- | --- | --- | --- | --- | --- |
| Speeding Up (L02A1-1) | Graphs position, velocity and acceleration vs. time. | Motion sensor 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis feature, statistics feature, graphical fit and statistics feature to find average acceleration from velocity- time and acceleration-time graphs. | Three sets of graph axes with lines. Position: 0-2 m Velocity: -1 to +1 m/s Accel.: -2 to +2 m/s2 Time 0-5 sec |
| Slowing Down (L02A3-1) | Graphs position, velocity and acceleration vs. time. | Motion sensor 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | NA | Two sets of graph axes with lines. Velocity: -1 to +1 m/s Accel.: -2 to +2 m/s2 Time 0-5 sec |
| Measuring Force (L03A1-2a) | Graphs force vs. time. | Force sensor 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis feature to read data from graph. | One set of graph axes with line. Force -10 to +10 N Time 0-10 sec |
| Rubber Bands (L03A1-2b) | Graphs force vs. number of rubber bands pulled. Force data are continuously measured and can be kept as desired. | Force sensor. Event mode--keep force values only when desired. | NA | Use analysis feature and fit routine to find relationship between force and number of rubber bands pulled. | One set of graph axes with line. Force -10 to +10 N # Rubber Bands: 0-5 |
| Calibrating Force (L03A1-4) | Graphs force vs. time | Force sensor 20 points/sec | NA | Use analysis feature to read data from graph | One set of graph axes with line. Force -10 to +10 N Time 0-10 sec |

| Motion and Force (L03A2-1) | Graphs velocity, force and acceleration vs. time. | Motion sensor and force sensor. 20 points/sec | NA | NA | Three sets of graph axes with lines. Velocity: -1 to +1 m/s Force: -2 to +2 N Accel.: -2 to +2 m/s2 Time 0-5 sec |
| --- | --- | --- | --- | --- | --- |

| Speeding Up Again (L03A2-2) | Graphs velocity, acceleration and force vs. time. | Motion sensor and force sensor. 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis feature and statistics feature to find average force and average acceleration from graph. | Three sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -2 to +2 m/s2 Force: -2 to +2 N Time 0-3 sec |
| --- | --- | --- | --- | --- | --- |
| Acceleration vs. Force (L03A2-5) | Graphs avg. acceleration vs. avg. force. Data are entered manually. | NA | NA | Use graphical fit routine to find relationship between average force and average acceleration. | Table for entering data manually. One set of graph axes with line. Acceleration and force ranges as appropriate. |
| Slowing Down Again (L04A1-1) | Graphs velocity, acceleration and force vs. time. | Motion sensor and force sensor Push is positive 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | NA | Three sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -2 to +2 m/s2 Force: -2 to +2 N Time 0-3 sec |
| Cooperating Fan Units (L04A2-2a) | Graphs force vs. time. | Force sensor 20 points/sec | NA | Use analysis feature and statistics feature to find average force from graph. | One set of graph axes with line. Force: -2 to +2 N Time 0-10 sec |
| Velocity and Acceleration (L04A2-2b) | Graphs velocity and acceleration vs. time. | Motion sensor 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis feature and statistics feature to find average accelerations from graph. | Three sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -2 to +2 m/s2 Time 0-5 sec |

| Dueling Fan Carts (L04A3-1) | Graphs velocity and acceleration vs. time. | Motion sensor 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | NA | Two sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -2 to +2 m/s2 Time 0-5 sec |
| --- | --- | --- | --- | --- | --- |
| Once a Pull (L04E3-2) | Graphs velocity, acceleration and force vs. time. | Motion sensor and force sensor. 20 points/sec | NA | NA | Three sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -2 to +2 m/s2 Force: -2 to +2 N Time 0-5 sec |

| Acceleration and Mass (L05A1-1) | Graphs velocity, acceleration and force vs. time. | Motion sensor and force sensor. 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis feature and statistics feature to find average forces and average accelerations from graphs. | Three sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -2 to +2 m/s2 Force: -2 to +2 N Time 0-3 sec |
| --- | --- | --- | --- | --- | --- |
| Avg. Acceleration vs. Mass (L05A1-4) | Graphs avg. acceleration vs. mass. Data are entered manually. | NA | NA | Use graphical fit routine to find relationship between average acceleration and mass. | Table for entering data manually. One set of graph axes with line. Acceleration andmass ranges as appropriate. |
| Acceleration with 1.0 N (L05A2-2) | Graphs velocity, acceleration and force vs. time. | Motion sensor and force sensor 20 points/s | NA | Use analysis feature and statistics feature to find average forces and average accelerations from graphs. | Three sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -2 to +2 m/s2 Force: -2 to +2 N Time 0-3 sec |
| Falling Ball (L06A1-1) | Graphs velocity and acceleration vs. time. | Motion sensor Distance away is negative 30 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis and statistics features to find average acceleration from graph. | Two sets of graph axes with lines. Velocity: -5 to +5 m/s Accel.: -15 to +15 m/s2 Time 0-2 sec |
| Inclined Ramp (L60A1-5) | Graphs velocity and acceleration vs. time. | Motion sensor Distance away is negative 20 points/sec | NA | Use analysis and statistics features to find average acceleration from graph | Two sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -4 to +4 m/s2 Time 0-5 sec |

| Action of Friction (L07A1-1) | Graphs velocity, acceleration and force vs. time. | Motion sensor and force sensor 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis feature and statistics feature to find average forces and average accelerations from graphs. | Three sets of graph axes with lines. Velocity: -2 to +2 m/s Accel.: -2 to +2 m/s2 Force: -2 to +2 N Time 0-5 sec |
| --- | --- | --- | --- | --- | --- |
| Static and Kinetic Friction (L07E1-2) | Graphs force and velocity vs. time. | Motion sensor and force sensor 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis and statistics features to find average forces from graph | Two sets of graph axes with lines. Force: -2 to +2 m/s Velocity: -0.5 to +0.5 m/s Time 0-5 sec |

| Tug-of-War (L07A2-1) | Graphs force sensors 1 and 2 vs. time. | Two force sensors 20 points/sec Sign of force sensor 2 is reversed. | NA | NA | Two sets of graph axes with lines. Forces: -10 to +10 N Time: 0-10 sec |
| --- | --- | --- | --- | --- | --- |

| Tension Forces (L07A3-1) | Graphs force sensors 1 and 2 vs. time. | Two force sensors 20 points/sec Sign of force sensor 2 is reversed. | NA | NA | Two sets of graph axes with lines. Forces: -10 to +10 N Time: 0-10 sec |
| --- | --- | --- | --- | --- | --- |
| Tension and N2 (L07E3-3) | Graphs force and acceleration vs. time. | Motion sensor and force sensor 20 points/sec | NA | Use analysis feature and statistics feature to find average forces and average accelerations from graphs. | Two sets of graph axes with lines. Force: -5 to +5 N Accel.: -4 to +4 m/s2 Time 0-3 sec |
| Clay vs. Superball (L08A1-2) | Graphs force vs. time. | Force sensor only 5000 points/sec Push is positive Triggered mode | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis feature to find maximum forces from graphs. | One set of graph axes with line. Force: can be uncalibrated Time 0-0.2 sec |

| Impulse and Momentum (L08A2-2) | Graphs velocity and force vs. time | Motion sensor and force sensor 50 points/sec Velocity toward motion detector positive Push is positive | NA | Use analysis and statistics features to find the average velocities from graphs. Use integration routine to find area under force vs. time graph. | Two sets of graph axes with lines. Velocity -2 to +2 m/s Force: -10 to +10 N Time 0-2 sec |
| --- | --- | --- | --- | --- | --- |
| Collisions (L09A1-1) | Graphs force sensors 1 and 2 vs. time | Two force sensors 5000 points/sec Sign of one sensor reversed Triggered mode | NA | Use integration routine to find area under force vs. time graphs. | Two sets of graph axes with lines. Forces: -10 to +10 N Time 0-0.2 sec |
| Other Interactions (L09A1-2) | Graphs force sensors 1 and 2 vs. time | Two force sensors 20 points/sec Sign of one sensor reversed | NA | NA | Two sets of graph axes with lines. Forces: -10 to +10 N Time 0-5 sec |
| Inelastic Collisions (L09A2-1) | Graphs velocity vs. time | Motion sensor 20 points/sec | NA | Use analysis and statistics features to find the velocities from graphs. | One set of axes with line. Velocity -2 to +2 m/s Time 0-5 sec |
| Horizontal Motion (L10A1-1) | Graphs position and velocity vs. time. | Motion sensor 20 points/sec | NA | NA | Two sets of graph axes with lines. Position 0 to 2 m Velocity -1 to +1 m/s Time 0–5 s |
| Vertical Motion (L10A1-2) | Graphs position, velocity and acceleration vs. time. | Motion sensor 30 points/sec | NA | Use the fit routine to find relationship between velocity and time. | Three sets of graph axes with lines Position 0 to 2 m Velocity -1 to +1 m/s Acceleration -10 to +10 m/s2 Time 0–3 s |
| Projectile Motion (L10A2-1) | Video analysis file with movie Ball Toss | NA | NA | Use the modeling routine to find the equations for x position, x velocity, y position and y velocity vs. time. | x position, x velocity, y position and y velocity vs. time |

| Force and Work (L11A1-2) | Graphs force vs. time. | Force sensor only 20 points/sec | Use to display data from first part persistently on the screen for comparison with data from second part. | Use analysis and statistics features to find the average forces from the graphs. | One set of axes with line. Force -5 to +5 N Time 0-10 sec |
| --- | --- | --- | --- | --- | --- |
| Work in Lifting (L11A2-1) | Graphs velocity and force vs. time. Then switch to force vs. position. | Motion sensor and force sensor 20 points/sec | NA | Use analysis and statistics features to find the average force from the graph. Use the integration routine to find the area under the force vs. position graph. | Two sets of axes with line. Velocity -1 to +1 m/s Force -5 to +5 N Time 0-10 sec |
| Stretching Spring (L11E2-2) | Graphs force vs. position. | Motion sensor and force sensor 20 points/sec | NA | Use the integration routine to find the area under the force vs. position graph. | One set of axes with line. Force -5 to +5 N Position 0-2 m |
| Kinetic Energy (L11A3-2) | Graphs velocity and kinetic energy vs. time. | Motion sensor 20 points/sec | Calculated column for kinetic energy with mass entered manually | NA | Two sets of axes with lines. Velocity -1 to +1 m/s Kinetic energy 0 to 50 J Time 0-10 sec |

| Work-Energy (L11A3-3) | Graphs force and kinetic energy vs. position. | Motion sensor and force probe 20 points/sec | Calculated column for kinetic energy with mass entered manually | Use the integration routine to find the area under the force vs. position graph. Use the analysis feature to find the kinetic energy from the graph. | Two sets of axes with lines. Force -5 to +5 N Kinetic energy 0 to 1 J Time 0-2 sec |
| --- | --- | --- | --- | --- | --- |

| Measuring Grav. Pot. E. (L12A1-1) | Graphs position and gravitational potential energy vs. time. | Motion sensor 20 points/sec | Calculated column for gravitational potential energy with mass entered manually | NA | Two sets of axes with lines. Position 0-2 m Gravitational potential energy 0 to 10 J Time 0-10 sec |
| --- | --- | --- | --- | --- | --- |
| Mechanical Energy (L12A1-2) | Graphs mechanical energy, kinetic energy and gravitational potential energy vs. position. | Motion sensor 20 points/sec | Calculated columns for mechanical energy, kinetic and gravitational potential energy with mass entered manually | NA | Two sets of axes with lines. Mechanical energy 0-20J Kinetic and gravitational potential energy on same axes -5 to 15 J Position 0-2 m |
| Inclined Ramp (L12A1-3) | Graphs mechanical energy, kinetic energy and gravitational potential energy vs. position. | Motion sensor 20 points/sec | Calculated columns for mechanical energy, kinetic and gravitational potential energy with mass entered manually | NA | Two sets of axes with lines. Mechanical energy 0-3J Kinetic and gravitational potential energy on same axes -0.5 to + 2.5 J Position 0-2 m |
| Spring Constant (L12A2-1) | Graphs force vs. position. | Motion sensor and force sensor Event mode--keep force and position values only when desired 20 points/sec | New prompted manual column to enter mass. | Use graphical fit routine to find relationship between force and position. | One set of axes with line. Force 0-2 N Position 0-1.5 m |
| Mechanical Energy Too (L12A2-2) | Graphs mechanical energy, kinetic energy and elastic potential energy vs. position. | Motion sensor 20 points/sec Velocity and acceleration data averaging to 3 points | Calculated columns for mechanical energy, kinetic and elastic potential energy with mass, spring constant and equilibrium position entered manually | Use the analysis and statistics features to find the equilibrium position from graph. | Two sets of axes with lines. Mechanical energy 0-0.05J Kinetic and elastic potential energy on same axes 0-0.05 J Time 0-5 sec |