SECTION L2:  LAB 2--changing Motion

General Introduction

In this lab, students explore motion with a constant acceleration (steadily changing velocity).  Graphs of position, velocity and acceleration will be displayed.  The relationship between velocity and acceleration in one dimension is studied.  Vector representation is developed further, with vectors used to represent velocities and accelerations.

Lab 2 Equipment and Supplies List

•	computer-based laboratory system

•	motion detector

•	RealTime Physics Mechanics experiment configuration files

•	cart with very little friction

•	smooth ramp or other level surface 2-3 meters long

•	fan unit attachment with batteries and dummy cells (or with a speed adjustment control)

Cart:  In order to get the smoothest possible acceleration graphs, considerable care must be taken in choosing a cart and a ramp.  The cart must have smooth wheels that do not rub or bind.  The best results are obtained by using dynamics carts with roller bearing wheels such as the Vernier CART-P or CART-S, or the PASCO # ME-9430, 9454 or 6950.

The carts from both Vernier and PASCO have very little friction, are sturdy and are relatively inexpensive.  Both companies have a friction pad attachment available which converts the low friction carts to ones with adjustable friction.  (See below.)

Ramp: Tracks with grooves that match the carts are also available from both Vernier and PASCO. With the newer motion detectors that have a shorter “dead” distance (i.e., minimum distance that an object must be away from the detector to have its position measured accurately) the meter or so length tracks are quite adequate. This laboratory does not strictly need a ramp since it is done on a level surface.   A smooth table top with a smooth (e.g., Formica) top and a clear distance of about a meter or so will work just fine.  However, many tables are too short or have too many nicks and scratches.  Pushing two tables together will not work because of the crack in the middle.  Having ramps will be advantageous for Labs 3 and 5, to provide a longer falling distance for the hanging mass in the modified Atwood's machine.  Ramps will also make it easier to investigate inclined planes.

An alternative to purchasing tracks is to purchase 3’ or 6’ door thresholds.  (These can be purchased for $20-30.)  Bring a cart with you when you go to purchase them to check that they will work.  Actually, only one set of wheels needs to run in a groove.  It will likely be necessary to tape over the screw holes so that they are not seen by the motion detector.

If instead you choose to build ramps, they should be about 1-2 m long and 20 - 30 cm wide, with a very smooth top surface.  A design that works well is shown in Figure L2-1.  It uses 3/4" plywood with one furniture grade side.  From one 4'x8' sheet you can make three ramps.  Cut three 12" wide strips 8' long and six, 2" strips to glue and screw edgewise to the 12" strips.  Without these strips, the plywood will bow too much when supported at two points.  Glue plastic coated paneling or Formica on the board to provide a smooth surface for the carts.

Figure L2-1: Possible construction of a ramp.

Fan Unit: fan accessories that on the dynamics carts are available from Vernier (FAN-VDS) and PASCO (ME-9491).  A Fan Cart is available from PASCO (#ME-6977).  The cart has the same low friction wheels as the PASCO dynamics cart.  However, the fan cart does not have collision bumpers or a friction pad assembly, and may not be as convenient for other standard experiments (e.g., collisions) as a dynamics cart.

It is possible to construct fan units for use with the dynamics carts.  The design described below and pictured in Figure L2-2 is adapted from a fan unit described by Robert Morse in his October, 1993 paper in The Physics Teacher (vol. 31, pp. 336-438).

Figure L2-2:  Fan Unit Design

You will need the following major parts for each fan unit:

•	6.5 cm long piece of 6.5 cm cross-section PVC downspout

•	DC motor with a no-load speed of about 8000 rpm (Radio Shack #273-223 works)

•	12.5 cm nylon propeller (e.g., #858 from Cox Hobbies, Corona, CA)

•	Battery holder for 4 AA batteries (e.g., Radio Shack #270-391)

•	SPST push-button switch (e.g., Radio Shack #275-1565 ).  (Substituting a DPDT switch will make it possible to reverse the thrust of the fan unit.)

Cut one side off the downspout, leaving a U-shaped piece that will grasp onto the top of the cart.  (When in use, the fan unit should always be taped onto the cart.  This precaution will prevent it from flying off when the cart is brought to a sudden stop, such as by colliding with a bumper.)  The motor is fastened to the top of the PVC section, and the switch is inserted in a hole drilled through the top.  The battery holder is fastened to the side with self-tapping sheet metal screws.   Finally, the most difficult problem is fastening the propeller to the motor shaft.  The best, i.e., most permanent solution is to machine a piece of metal with a hole and set screw on one end to fasten to the motor shaft and a threaded hole on the other end for fastening the propeller with a machine screw.  This is labor intensive, but we have not found a satisfactory method of gluing or pressure-fitting the propeller to the motor shaft.

Rechargeable batteries are a must!  The batteries should be charged up before each student work period, and students should be warned to only turn the motors on during the time when actual measurements are made.

The battery voltage can be varied by replacing one or more AA cells with dummy cells cut from 0.5-in diameter aluminum rod.  Another option would be to include a potentiometer in the circuit to make the voltage applied to the motor variable.

Great care should also be taken to keep the fan units from falling on the floor.  The impact can bend the brushes in the motor and stop it from working.

A modified Atwood's machine (pulley and falling mass) can be used in place of the fan unit, and works well.  (See Equipment and Materials in the Lab 3 write-up for more directions.)  It is important in this case to provide at least a 1 m distance for the mass to fall by elevating the ramp or using two pulleys.  However, students find the fan units more interesting to study.

Remember that the cart must never be closer than the minimum specified distance from the motion detector.

Investigation 1:  velocity and acceleration graphs

Equipment and Supplies

•	computer-based laboratory system

•	motion detector

•	RealTime Physics Mechanics experiment configuration files

•	cart with very little friction

•	smooth ramp or other level surface 1.2-2.5 meters long

•	fan unit attachment with batteries and dummy cells (or with a speed adjustment control)

Overview

Here students look at the relationship between acceleration and velocity for cases in which the cart is speeding up moving away from the motion detector with a constant acceleration.

Activity 1-1:  Speeding Up

Figure L2-3 shows graphs of position, velocity and acceleration vs. time for a low friction cart moving away from the motion detector with a constant acceleration, as in Activity 1-1.

If your position graph levels off and the velocity and accelerations fall to zero at the same instant, then the motion detector is likely seeing something other than the cart.  Use the analysis feature of the software to locate where this object is, and move it or tilt the motion detector slightly away from it.

If your velocity and especially your acceleration graphs are bumpier than those in Figure L2-3, several things might be wrong:

1.	The track is not smooth, and the bumps represent real characteristics of the motion of the cart.

2.	The bearings of the cart's wheels are bad, and they are causing the acceleration to be non-constant.

3.	The fan blade is extending beyond the end of the cart, and the motion detector is intermittently seeing the fan blade and the cart.

If you have eliminated (1) and (2), and moved the fan unit back, and the graphs are still bumpy, try taping a stiff piece of cardboard a few inches high to the front of the cart to act as a flag.  This should improve the graphs immensely.

| Figure L2-3: Position-, velocity- and acceleration-time graphs for a low-friction cart with a fan unit moving away from the motion detector with a constant acceleration as in Activity 1-1. |  |
| --- | --- |

Question 1-1:  Instead of being a straight line, this position-time graph curves upwards.  That is, the slope increases as the cart moves along.  This is because the velocity is increasing.

Question 1-2:  The velocity is always a positive number, indicating motion in the positive x direction.

Question 1-3:  The line on the velocity-time graph has a positive slope, indicating that the magnitude of the velocity (speed) is increasing.  Motion with a constant velocity would be represented by a horizontal line (parallel to the time axis).

Question 1-4:  The acceleration is positive.  The change in velocity (v2 - v1) in a time interval is always positive, and therefore the acceleration is positive.

Question 1-5:  The velocity increases at a steady rate.  This is indicated by the constant slope (constant rate of change of velocity) of the velocity-time line.

Question 1-6:  The acceleration is essentially constant as the cart speeds up.  This indicates a constant rate of change of velocity with time as observed on the velocity-time graph.

Question 1-7:

The vector points toward the right, and therefore the acceleration-- which is the change of velocity divided by the time interval (always positive)--is positive.  This agrees with Question 1-4.

Activity 1-2:  Speeding Up More

Be sure that the thrust of the fan unit is significantly larger, by doubling the number of batteries (or turning the dial to the maximum speed if you have a fan unit with a speed control).

Question 1-9:  Answer depends on student results.  The larger the acceleration, the larger the slope of the line on the velocity-time graph.

Question 1-10:  The larger the acceleration, the higher the line is above the time axis.

investigation 2:  Measuring acceleration

Equipment and Supplies

•	computer-based laboratory system and data files saved from Investigation 1

Overview

Here students explore the calculation of average acceleration from velocity-time and acceleration-time graphs.  The data and graphs from Investigation 1 will be used.  This is analogous to the calculations of velocity from position-time and velocity-time graphs in Lab 1, Investigation 3.

Activity 2-1:  Velocity and Acceleration of a Cart That is Speeding Up

It is important that students take their data for velocity and acceleration during the time interval when the cart was accelerating.  The two methods for calculating average acceleration are equivalent and should give the same value.

Question 2-1:  The acceleration is positive.  This is what is expected since the cart was moving away from the motion detector and speeding up.

Question 2-2:  For a constant acceleration the values should agree.

Activity 2-2:  Speeding Up More

Question 2-3: For a constant acceleration the values should agree.

Question 2-4:  This acceleration was larger.

Extension 2-3:  Using Statistics and Fit to Find the Average Acceleration

You have previously used these two mathematical methods built into the software in Extension 3-3 in Lab 1. Finding the mean value is the same as reading a number of values from the acceleration-time graph, only the software averages all values within the time interval selected.  Finding the slope of the line which fits the velocity-time graph is equivalent to taking two points and calculating the change in velocity divided by the change in time, but the slope can be found more accurately in this way.

Question E2-5:  The values should be similar.

Question E2-6:  The equation for velocity as a function of time is v = b + mt.  The value of b is the velocity of the cart at the moment that the motion detector began detecting (t = 0).

Question E2-7:  The two values should agree.

Investigation 3:  slowing down and speeding up

Equipment and Supplies

•	computer-based laboratory system

•	motion detector

•	RealTime Physics Mechanics experiment configuration files

•	cart with very little friction

•	smooth ramp or other level surface 1.2-2.5 m long

•	fan unit attachment with batteries

Overview

Here students look at the relationship between acceleration and velocity for cases in which the cart is slowing down and/or moving towards the motion detector.  These tend to be considerably more difficult for students.  It is essential that they master these concepts before moving on to dynamics and Newton’s Laws of Motion.

Activity 3-1:  Slowing Down

Figure L2-4 shows velocity and acceleration graphs for a low-friction cart with a fan unit moving away from the motion detector and slowing down at a steady rate as in Activity 3-1.

| Figure L2-4: Velocity and acceleration graphs for a low- friction cart with a fan unit moving away from the motion detector and slowing down at a steady rate as in Activity 3-1. |  |
| --- | --- |

Letters A, B, C and D are the times when A) the push started, B) the push ended, C) only the force of the fan is acting on the cart, and D) the cart stopped moving.

Question 3-1:  Answer depends on student data.  The sign of the slope of the velocity-time graph gives the sign of the acceleration.  Since the line in the velocity-time graph in Figure L2-4 is sloped downward, the slope is negative and the acceleration is negative.

Question 3-2:  The sign of the acceleration is determined by the values of the acceleration on the graph.  Since the horizontal line in the acceleration-time graph in Figure IV-4 is below the time axis, it has negative values, and the acceleration is negative.

Question 3-3:  Answer depends on student data.  The velocity is positive but decreasing in magnitude.  Therefore the change in velocity, v = v2 - v1 is negative, and the acceleration is negative.

Question 3-4:

The sign of the acceleration is negative.  This agrees with Question 3-3.

Question 3-6:  If the object is moving in the positive direction, its velocity is positive.  Then, its acceleration is positive if it is speeding up, and negative if it is slowing down.

Activity 3-2:  Speeding Up Toward the Motion Detector

Figure L2-5 shows velocity and acceleration graphs for a low-friction cart with a fan unit moving toward the motion detector and speeding up at a steady rate as in Activity 3-2.

| Figure L2-5: Velocity and acceleration graphs for a low- friction cart with a fan unit moving toward the motion detector and speeding up at a steady rate as in Activity 3-2. |  |
| --- | --- |

Question 3-7:  The velocity is always negative.

Question 3-8:  The acceleration is negative.  Answer depends on student data.  The velocity is negative and increasing in magnitude.  Therefore the change in velocity, v = v2 - v1 is negative, and the acceleration is negative.

Question 3-9:  When an object is speeding up, the acceleration is in the same direction as the velocity.

Question 3-10:

The sign of the acceleration is negative.  This agrees with Question 3-8.

Question 3-12:  When an object is moving in the negative direction, its acceleration is positive if it is slowing down and negative if it is speeding up.

Question 3-13:  The acceleration should be positive. The velocity is negative and decreasing in magnitude.  Therefore the change in velocity, v = v2 - v1 is positive, and the acceleration is positive.

Question 3-14:

Extension 3-3:  Graphing Slowing Down Toward the Motion Detector

Question E3-16:  Answer depends on student data.

Activity 3-4:  Reversing Direction

Figure L2-6 shows velocity and acceleration graphs for a low-friction cart with a fan unit moving away from the motion detector, slowing down, coming to rest instantaneously, and then moving toward the motion detector, speeding up as in Activity 3-2.

| Figure L2-6: Velocity and acceleration graphs for a low- friction cart with a fan unit moving away from the motion detector, slowing down, coming to rest instantaneously, and then moving toward the motion detector, speeding up as in Activity 3-3. |  |
| --- | --- |

Question 3-17:  Letters A, B, C and D are the times when A) the push started, B) the push ended, C) the turning point, and D) the cart stopped moving.

Question 3-18:  The velocity was zero for one instant at the turning point.  Immediately before and immediately after this instant, the cart was moving.

Question 3-19:  The acceleration of the cart is negative throughout the motion, including the instant when it turned around.

Question 3-20:  The acceleration is calculated as the change in velocity divided by the change in time.  Calculating the change in velocity involves taking the difference between the velocities at two times.  Just before the turning point, the velocity is positive, and just after, the velocity is negative.  Therefore, v is always negative even though one of the velocities used in its calculation is zero.

Question 3-21:  The sections of the two sets of graphs from Figure L2-6 and L2-5 look the same.

Extension 3-5:  Sign of Push and Stop

Question E3-22:  The signs of the acceleration for the initial push and for the stop are both positive.  In the push, the cart is moving away from the motion detector and speeding up.  In the stop, the cart is moving towards the motion detector and slowing down.  These both result in a positive acceleration.

Question E3-23:  The ball always has a negative acceleration, on the way up, at its turning point and on the way down.  The ball is accelerated by the Earth’s gravitational pull downward.

HOMEWORK for Lab 2:  Changing motion

| 1. An object moving along a line (the + position axis) has the acceleration-time graph on the right. |  |
| --- | --- |

a.	Describe how the object might move to create this graph if it is moving away from the origin.

Move away from the origin with a steadily increasing velocity.

b.	Describe how the object might move to create this graph if it is moving toward the origin.

Move toward the origin with a steadily decreasing velocity.

| c. Sketch with a solid line on the axes on the right a velocity-time graph that goes with the motion described in (a). d. Sketch with a dashed line on the axes on the right a velocity-time graph that goes with the motion described in (b). |  |
| --- | --- |

| 2. How would an object move to create each of the three labeled parts of the acceleration-time graph shown below? a: Away from origin with steadily increasing speed or towards the origin with steadily decreasing speed. |  |
| --- | --- |

b:  Move without any change in velocity.

c:  Move away from the origin with steadily decreasing speed, or towards the origin

with steadily increasing speed.

| 3. Sketch on the right a velocity-time graph which might go with the acceleration-time graph in question (2). |  |
| --- | --- |

4.  For each of the velocity-time  graphs on the left, sketch the shape of the acceleration-time graph that goes with it.

5.	A car can move along a line (the + position axis).  Sketch  velocity-time and acceleration-time graphs which correspond to each of the following descriptions of the car's motion.

6.  The following is a velocity-time graph for a car.

What is the average acceleration of the car?  Show your work below.

a = v/t = (4 - 1)/(10 - 0) = 0.3 m/s

7.	A car moves along a line (the + position axis).  Fill in the table below with the sign (+ or -) of the velocity and acceleration of the car for each of the motions described.

8. 	A ball is tossed in the air.  It moves upward, reaches its highest point and falls back downward.  Sketch a velocity-time and an acceleration-time graph for the ball from the moment it leaves the thrower's hand until the moment just before it reaches her hand again.  Consider the positive direction to be upward.

9.	For each of the position-time graphs shown, sketch below it the corresponding velocity-time and acceleration-time graphs.

10.	Each of the pictures below represents a car driving down a road.  The motion of the car is described.  In each case, draw velocity and acceleration vectors above the car which might represent the described motion.  Also specify the sign of the velocity and the sign of the acceleration.  (The positive position direction is toward the right.)