appendix a:  realtime physics heat anD THERMODYNAMICS EXPERIMENT CONFIGURATION FILES

Listed below are the settings in the Experiment Configuration Files used in these labs.  These files are available from Vernier Software for the Logger Pro software and from PASCO Scientific for Data Studio. They are listed here so that the user can set up files for any other compatible hardware and software package.

| Experiment File | Description | Data Collection | Data Handling | Analysis | Display |
| --- | --- | --- | --- | --- | --- |
| Digital Readouts (L1A1-1) | Displays digital readings of temp. sensors 1 and 2. | Temp. sensors 1 and 2 Digital display of inputs | NA | NA | Digital display only |
| Reaching Equilibrium (L01A1-2) | Displays and graphs temp. sensors 1 and 2 vs. time. | 10 points/sec Temp. sensors 1 and 2 Digital display of inputs | NA | NA | One set of graph axes with lines. Temp.: 0-30°C Time: 0-240 sec |
| Cooling Water (L01A2-1) | Displays temp. sensors 1 and 2, and graphs temp. sensor 1 vs. time. | 10 points/sec Temp. sensors 1 and 2 Digital display of inputs | NA | NA | One set of graph axes with lines. Temp.: 0-100°C Time: 0-10 min |
| Temp. Change and Heat Transfer (L1A2-2) | Displays and graphs temp. sensors 1 and 2 vs. time. | 10 points/sec Temp. sensors 1 and 2 Digital display of inputs | NA | NA | One set of graph axes with lines. Temp.: 0-100°C Time: 0-5 min |
| Keeping Hot (L01A3-1) | Displays and graphs temp. sensor 1 only vs. time. Enables heat pulser to transfer heat pulses. | 10 points/sec Temp. sensor 1 only Digital display of input Heat pulser enabled, 2 sec pulse length | NA | NA | One set of graph axes with line. Temp.: 70-90°C Time: 0-120 sec |

| Heating Water (L01E3-2) | Displays and graphs temp. sensor 1 only vs. time. Enables heat pulser to transfer heat pulses. | 10 points/sec Temp. sensor 1 only Digital display of input Heat pulser enabled, 5 sec pulse length | Use to display data from first part persistently on the screen for comparison with data from second part | May use analysis feature to read data from graphs after collected | One set of graph axes with line. Temp.: 10-40°C Time: 0-150 sec |
| --- | --- | --- | --- | --- | --- |
| Mech. and Elect. Work (L02A1-1) | Displays and graphs temp. sensor 1 only vs. time. | 10 points/sec Temp. sensor 1 only Digital display of input | Use to display data from first part persistently on the screen for comparison with data from second part | May use analysis feature to read data from graphs after collected | One set of graph axes with line. Temp.: 20-40°C Time: 0-180 sec |
| Heating Water (L02A2-1) | Displays and graphs temp. sensor 1 only vs. time. Enables heat pulser to transfer heat pulses. | 10 points/sec Temp. sensor 1 only Digital display of input Heat pulser enabled, 5 sec pulse length | Use to display data from first part persistently on the screen for comparison with data from second part | May use analysis feature to read data from graphs after collected | One set of graph axes with line. Temp.: 20-40°C Time: 0-180 sec |
| Keeping Hot (L03A1-1) | Displays and graphs temp. sensor 1 only vs. time. Enables heat pulser to transfer heat pulses. | 10 points/sec Temp. sensor 1 only Digital display of input Heat pulser enabled, 2 sec pulse length | NA | NA | One set of graph axes with line. Temp.: 70-90°C Time: 0-5 min |
| Math. Relation (L03E1-2) | Displays data table and graph for number of pulses and temperature difference data that are entered manually | NA | New manual columns of data for # Heat Pulses and Temp. Diff. | Use graphical fit options to find experimental relationship between number of heat pulses and temp. diff. | One set of graph axes with line. # Heat Pulses: 0-20 Temp. Diff.: 0-80°C |
| Cooling Down (L03A2-1) | Displays and graphs temp. sensors 1 and 2 vs. time. | 10 points/sec Temp. sensors 1 and 2 Digital display of inputs | NA | May use analysis feature to read data from graphs after collected | One set of graph axes with lines. Temp.: 40-90°C Time: 0-5 min |
| Radiation (L03A3-2) | Displays and graphs temp. sensor 1 only vs. time. | 10 points/sec Temp. sensor 1 only Digital display of input | Use to display data from first part persistently on the screen for comparison with data from second part | NA | One set of graph axes with lines. Temp.: 20-35°C Time: 0-5 min |
| Ice to Water (L04A1-1) | Displays and graphs temp. sensor 1 only vs. time. Enables heat pulser to transfer heat pulses. | 10 points/sec Temp. sensor 1 only Digital display of input Heat pulser enabled, 10 sec pulse length | NA | May use analysis feature to read data from graphs after collected | One set of graph axes with line. Temp.: -10-30°C Time: 0-5 min |
| Water to Steam (L04A2-1) | Displays and graphs temp. sensor 1 only vs. time. Enables heat pulser to transfer heat pulses. | 10 points/sec Temp. sensor 1 only Digital display of input Heat pulser enabled, 10 sec pulse length | NA | May use analysis feature to read data from graphs after collected | One set of graph axes with line. Temp.: 10-110°C Time: 0-10 min |
| Measuring Pressure (L05A1-2) | Displays pressure sensor 1 only | Pressure sensor 1 only Digital display of input | NA | NA | No axes. Just digital display. |
| Pressure vs. Volume (L05A2-1) | Displays pressure and volume. Pressure is continuously measured while volume is entered manually after pressure value is kept. | 10 points/sec Pressure sensor 1 only Digital display of inputs Event mode--keep pressure values only when desired | New calculated column that displays pressure in 105 Pa [P(atm) x 1.01325]. Volume in cm3 entered manually. | Use graphical fit options to find experimental relationship between pressure and volume. | One set of graph axes with line. Press.: 0-8 x 105 Pa Volume: 0-24 cm3 |

| Measuring P and T (L05A2-2) | Displays pressure and temperature. Both are continuously measured and can be kept as desired. | 10 points/sec Pressure sensor 1 and temp. sensor 2 Digital display of inputs Event mode--keep pressure and temp. values only when desired | New calculated column that displays pressure in 105 Pa [P(atm) x 1.01325]. New calculated column that displays temp. in K [T(°C)+273.15] | Use graphical fit options to find experimental relationship between pressure and temp. | One set of graph axes with line. Press.: 0.6-1.2 x 105 Pa Temp.: 270-370 K |
| --- | --- | --- | --- | --- | --- |
| Measuring V and T (L5A2-3) | Displays volume and temperature. Temperature and pressure are continuously measured while volume is entered manually after these values are kept. | 10 points/sec Pressure sensor 1 and temp. sensor 2 Digital display of inputs Event mode--keep pressure and temp. values only when desired | New column that displays pressure in 105 Pa [P(atm) x 1.01325]. New calculated column that displays temp. in K [T(°C)+273.15] Volume in cm3 entered manually. | Use graphical fit options to find experimental relationship between volume and temp. | One set of graph axes with line. Volume: 25-55 cm3 Temp.: 270-320 K |
| Single Particle Movie (L05E3-1) | Digital movie that shows a single particle in a box. Need movie viewer software like Apple Movie Player to view. |  |  |  |  |
| Syringe Movie (L05E3-2) | Digital movie that shows particles in a syringe with a moving plunger. Need movie viewer software like Apple Movie Player to view. |  |  |  |  |

| Pressure and Temperature (L6A3-2) | Displays pressure, temperature and volume. Pressure and temperature are continuously measured while volume is entered manually after pressure value is kept. | 10 points/sec Pressure sensor 1 and temperature sensor 2 Digital display of inputs Event mode--keep pressure values only when desired | New calculated column that displays pressure in 105 Pa [P(atm) x 1.01325]. New calculated column that displays temp. in K [T(°C)+273.15] Volume in cm3entered manually. | Use graphical fit options to find area enclosed in pressure vs. volume cycle. | One set of graph axes with line. Press.: 0.9-1.1 x 105 Pa Volume: 24 - 40 cm3 |
| --- | --- | --- | --- | --- | --- |