SECTION L6: LAB 6—HEAT ENGINES

General Introduction

These investigations introduce students to the thermodynamics applicable to the operation of heat engines. Such concepts as cyclic process, working medium, hot and cold reservoirs, adiabatic compression, P-V diagram, work done by a gas, efficiency and Carnot efficiency are explored. The operation of two actual heat engines—one constructed of a rubber band and one constructed of a glass syringe—are explored, with the latter examined quantitatively.

Lab 6 Equipment and Supplies List

• 	large rubber band,  5 cm x 10 cm

• 	mass pan

• 	two 10-kg masses

• 	table clamp with long rod, short support rod, and right angle clamp

• 	heat gun (or 1500 W hair dryer with Styrofoam housing around rubber band to keep heat energy from being transferred to the surroundings)

• 	fire syringe

• 	ruler

• 	glass thermometer

• 	tiny piece of tissue paper, approximately 0.5 mm on a side

•	safety gloves and goggles

• 	10-cm3 low friction glass syringe with ring stand support

•	several lengths of Tygon tubing

•	25-ml flask with one hole rubber stopper

•	2 insulated (e.g., Styrofoam) containers (to use as reservoirs)

•	ruler

•	50-g mass

•	hot water (about 80-90°C)

•	ice water

•	computer-based laboratory system

•	pressure sensor

•	temperature sensor

•	RealTime Physics Heat and Thermodynamics experiment configuration files

Fire syringe: Fire syringes are available from most physics equipment vendors.  They consist of a glass tube with a plunger that has a triple o-ring seal. The whole syringe is housed in a plastic tube with an air layer that helps insulate it from its surroundings.

Glass syringe: The essential element of this engine is a low friction, 10 cm3 glass syringe, with a luer-lock connector, that is commonly found in chemistry departments, and which may be purchased relatively inexpensively (around $20) from science equipment supply companies. As long as the plunger and cylinder are clean, the operation of this syringe will be with very low pressure. If the plunger starts to stick, clean it in distilled water and detergent and rinse in distilled water. The only drawback of this piece of apparatus is that it is very breakable. Extreme care must be taken that the plunger does not fall out of the syringe.

Alternatively, the more expensive PASCO Heat Engine/Gas Law Apparatus (TD-8572) will work very well in this experiment in place of the glass syringe.

Investigation 1: Heat engines and cycles

Equipment and Supplies

• 	large rubber band,  5 cm x 10 cm

• 	mass pan

• 	two 10-kg masses

• 	table clamp with long rod, short support rod, and right angle clamp

• 	heat gun (or 1500 W hair dryer with Styrofoam housing around rubber band to keep heat energy from being transferred to the surroundings)

Overview

This example of a heat engine depends on the property of rubber that it contracts when it is heated. It has many of the features of a real engine: cyclic nature, need for a heat energy input, necessity of a cooling part of the cycle and release of waste heat energy.

Activity 1-1:  The Rubber Band as a Heat Engine Medium

A good explanation of the contraction of the rubber band when it is heated is found in the Feynman Lectures on Physics, page 44-1 ff. It has to do with the structure of rubber which consists of long (polymer) chains are cross-linked. As a rubber band is stretched, these chains tend to line up parallel to each other, but the constant collisions of molecules in adjacent chains tends to kink up the chains again. When the rubber is heated, the vigor of these collisions increases, and there is a tendency for the chains to pull in and for rubber to contract.

Significant heating is necessary. If you don’t have a powerful heat gun, then it will be necessary to build a crude structure from Styrofoam around the rubber band to trap the heat energy from the hair dryer.

Question 1-1: The rubber band contracts a significant amount, and lifts the weight.

Question 1-2: You need to heat the rubber band.

Question 1-3: You need to push it onto the conveyor belt.

Question 1-4: The rubber band must stretch again so that the lifter platform is at the level of the lower conveyor belt. This means that the rubber band must cool down to its original temperature.

Question 1-5: Move can from filler conveyor belt onto lifter. Heat rubber band. Move from lifter onto packer conveyor belt. Cool rubber band. Start again with new can.

Question 1-6: Heat energy is transferred to the engine (rubber band) when the rubber band is heated by the heat gun. Work is done as the rubber band contracts and lifts the can. Heat energy is transferred from the engine (rubber band) as the rubber band cools down to complete the cycle.

Question 1-7: On a hot day, the rubber band could never cool down, and would never move down to the level of the feeder conveyor belt. A cycle would be impossible.

Question 1-8: Some of the heat energy is transferred to the rubber band’s surroundings as the rubber band cools down. It appears that only part of the heat energy transferred to the rubber band by the heat gun is converted to useful work in lifting the can.

nvestigation 2:  Adiabatic compression of gases

Equipment and Supplies

• 	fire syringe

• 	ruler

• 	glass thermometer

• 	tiny piece of tissue paper, approximately 0.5 mm on a side

•	safety gloves and goggles

Overview

Here students look in some detail at the ideas of adiabatic processes, and the energy balances in expansions, compressions and cyclic processes.

Extension 2-1:  The Fire Syringe and the Rapid, Adiabatic Compression of Air

It is possible to quickly compress the air in a fire syringe to a fraction of its original volume quickly, before any heat energy can be transferred out of the syringe.  This adiabatic process results in a large elevation of temperature.  A tiny piece of tissue paper, or better, some filings from the tip of a match head, will ignite at the temperature reached.

Question E2-1:  The piece of paper ignited.

Typical data are shown in the Table E6-1 below.

| Initial length of air column (cm) | 15.0 cm |
| --- | --- |
| Final length of air column (cm) | 0.5 cm |
| Inside diameter of the tube (cm) | 0.75 cm |
| Initial volume Vi (calculated) (cm3) | 2.11 |
| Final volume Vf (calculated) (cm3) | 0.07 |
| Initial temperature Ti (room temp.) (K) | 296 |
| Final temperature Tf (calculated) (K) | 1150 |

Question E2-2:  Tf2.5Vf = Ti2.5Vi  or [Tf/Ti]2.5= [Vf / Vi],

[Tf/296] 2.5= [2.11/0.07] = 30 and Tf = 1150 K.

Question E2-3:  T (°C) = (5/9)(451-32) = 234 °C

T (K) = T(°C) + 273 = 507 K.  1150 K is well above the flash point.

Question E2-4:  [Tf/295] 2.5= 10 and Tf = 741 K.

Question E2-5:  The final temperature by definition would be 295 K, since isothermal means with no temperature change.

Question E2-6:  In a slow compression, heat energy is transferred to the surroundings and the temperature inside the syringe does not rise. In the adiabatic compression, since heat energy cannot escape, the temperature inside the syringe rises rapidly.

Activity 2-2:  Work Done by an Expanding Gas

This activity is a review of calculation of the work done by (and on) a gas that expands and contracts.  There are no actual measurements involved.

Question 2-7:  W = PV

Question 2-8:  The work done is W = P (V2 – V1).  This is the same as the area under the P vs. V curve between 1 and 2.

Investigation 3:  The incredible mass lifting engine

Equipment and Supplies

• 	10-cm3 low friction glass syringe with ring stand support

•	several lengths of Tygon tubing

•	25-ml flask with one hole rubber stopper

•	2 insulated (e.g., Styrofoam) containers (to use as reservoirs)

•	ruler

•	50-g mass

•	hot water (about 80-90°C)

•	ice water

•	computer-based laboratory system

•	pressure sensor

•	temperature sensor

•	RealTime Physics Heat and Thermodynamics experiment configuration files

Overview

In this investigation students examine the operation of an actual heat engine, made from a low friction, 10 cm3 glass syringe, with a luer-lock connector.

Activity 3-1:  Description of the Engine Cycle

Question 3-1: The plunger moved downward. Since this happened quickly, it is possible it took place without any transfer of heat energy into or out of the syringe, in which case it was an adiabatic change.

Question 3-2: The platform rose. Since the pressure in the gas is the force per unit area, it is determined by the force exerted on the gas by the weight of the plunger and by atmospheric pressure exerted on the plunger. Neither of these changes during the process, so the pressure should remain constant.

Question 3-3: The plunger moved upward. As in 12, since this happened quickly, it is possible it took place without any transfer of heat energy into or out of the syringe, in which case it was an adiabatic change.

Question 3-4: The plunger moved down again. Again, the pressure is constant during this process since the mass remains on top of the plunger.

Question 3-5: It is probably somewhat less, since gas usually leaks out.

Question 3-6: If the volume, temperature and amount of gas are the same, then the pressure must be the same. The gas is back in its original state.

Activity 3-2:  Work Done by the Heat Engine

Here are sample data for this activity:

Inside diameter: 0.5 cm		Total length:  30 cm

V = (d2/4)L = (3.14)(0.5)2(30)/4 = 6 cm3.

Table 6-2a

| State of System | Volume of Air in Syringe (cm3) | Volume of Flask (cm3) | Volume of Tubing (cm3) | Volume Sensor (cm3) | Total Volume of Air (cm3) |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.0 | 25 | 6.0 | 1.0 | 35.0 |
| 2 | 2.0 | 25 | 6.0 | 1.0 | 34.0 |
| 3 | 9.0 | 25 | 6.0 | 1.0 | 41.0 |
| 4 | 10.5 | 25 | 6.0 | 1.0 | 42.5 |
| 1 | 3.2 | 25 | 6.0 | 1.0 | 35.2 |

Mass to be lifted: 0.05 kg

Height mass was raised: 0.041 m

Hot reservoir temperature: 357 K

Cold reservoir temperature: 277 K

Figure L6-1 shows a graph for a typical experiment.

| Figure L6-1: Pressure vs. volume graph of the heat engine cycle in Activity 3-2. |  |
| --- | --- |

Question 3-7:  They were nearly but not quite isobaric. The pressure changed somewhat.

Activity 3-3:  Calculating the Work Done by the Heat Engine

| Figure L6-2: Work done in the cycle calculated as the area enclosed in the cycle on the pressure vs. volume graph of the heat engine cycle in Activity 3-2. The cycle from Figure L6-1 has been closed by changing data point 5 so that it is the same as the initial state in data point 1. |  |
| --- | --- |

Method I:  The integration routine will give you incorrect results if the cycle is not closed. The best way to handle this is to change the last data point so that it is the same as the initial state of the system. This is acceptable, since it clearly only changes the area enclosed in the cycle by a small amount.

Figure L6-2 shows the determination of the area. The curve is most easily selected by selecting all of the data in the data table. The value is W =(0.206 x 105 Pa-cm3) = 0.21 (105)( 10-6 m3/cm3 ) = 0.021 J.

Method II:  The area is actually more like two triangles and a rectangle:

= 0.022 J.

The area is within 10% of the value from Method I.

Method III:  The cycle is re-drawn on the axes, and the number of boxes is counted up.  There are about 22 boxes enclosed by the cycle, each of which is 0.001 J.  Thus the work is 0.022 J, in agreement with the other two methods.

The mechanical work raising the 50 g mass through a height of 4.1 cm is given by W = mgy = (0.05)(9.8)(0.041)= 0.020 J.

Question 3-8: They are very nearly the same: 0.021J or 0.022 J vs. 0.020 J.  Depending on which calculation of the thermodynamics work is used, they are within 5-10% of each other.

Extension 3-4:  Efficiency of the Mass Lifting Heat Engine

1. n = PV/RT = (0.99 x 105)(35 x 10-6)/(8.31)(277) = 0.0015 mole

2.  Q = nCPT = (0.0015)(29.0)(81) = 3.5 J

3.	e = 100% (Wout/Qin) =  100%(.020/3.5) = 0.6%

4.	ec = 100% (1 -  TC/TH) = 100%(1-277/358) = 23%

Question E3-9: Not very efficient.  Only 0.6% of the input heat energy is converted to useful work.  99.4% is lost as waste heat energy.

Question E3-10: It is much smaller than the maximum possible efficiency, 0.6% vs. 23%.

Homework for Lab 6:  heat engines

1.	If 5.0 moles of air is at 0°C and 1.0 atm pressure (otherwise known as standard temperature and pressure, or STP for short), what is the volume occupied by the gas in m3?  (Treat air as an ideal gas.)

Using the ideal gas law;   PV = nRT;

;    V = 0.11 m3

2.	Suppose the gas in Question 1 is compressed isothermally until its volume is one-fifth as large.  What is the final pressure?  What is the final temperature?

Isothermal means T remains the same, Tf = Ti = 273.15K.  Again using the ideal gas law, and equating temperatures gives:

PiVinR=PfVfnR⇒Pf=PiViVf=PiVi15Vi   =  5Pi = 5 atm or 5.05x105 Pa

3.	The air in Question 1 is instead compressed adiabatically until its volume is one- fifth as large.  What is the final temperature?  What is the final pressure?

Air is a diatomic gas, and adiabatic means no heat flows in or out during compression.  We then use:

ViTi52=VfTf52; Tf=ViTi52Vf25  =  520K;  Pf = nRTf/Vf =                                                (5moles) ( 8.3J/mole-K) ( 520K)/.022m3 = 9.8x105Pa

4.	Explain macroscopically in terms of work and energy considerations why the temperatures in Questions 2 and 3 are different. What happens to the internal energy during the process in Question 2? What happens to the internal energy during the process in Question 3?

In (2), internal energy does NOT increase. All the work on the system is converted to heat energy, Q, that is transferred to the surroundings resulting in no temperature change:            U = Q - W = 0;  Q = W.

In (3), no heat is transferred to or from the surroundings. The change in internal energy equals minus the work done by the system: U = -W (but W is negative, since it is actually done on the system).

5.	On a microscopic level, how does the motion of molecules change when the temperature of a gas increases? Explain microscopically, in terms of collisions of gas molecules with the rapidly moving piston, why the temperature of a gas rises in an adiabatic compression.

The average speed of the random motions of molecules in a gas increases as the temperature of the gas increases. In a collision with a moving piston, the speed of a molecule increases.  Therefore, the average speed of the molecules will increase, and the temperature will increase.

6.	The air in Question 1 is compressed isobarically until its volume is one-fifth as large.  How much work was done?  Is the work done positive or negative?

The change in volume would be: V = Vf – Vi = 1/5 Vi – Vi = -4/5 Vi = -0.088m3

W = PV = 1.01x105Pa x (-0.088m3) = -8.9x103 J, work is negative—done on the system.

7.	Sketch the process in Question 6 on the axes below, and explain how to calculate the work done from the graph.

|  | Work would be the area under the line: W = P x (V2 - V1) It would be negative because volume decreases, V2 < V1 |
| --- | --- |

8.	Calculate the work done in the process from state 1 to state 2 shown on the P-V diagram below.  (Note:  1 MPa = 106 Pa.)  Is the work positive or negative? Explain. What would the work be if the process went from 2 to 1?

|  | The area under the curve can be estimated by adding the average height of each column (.e.g., “A”) multiplied by .005 m3: 6,600 Pa m3 = 6,600 J The work would be negative going from 2 to 1, as the volume would decrease for each column. |
| --- | --- |

9.	Suppose 1.5 moles of air are carried through the cyclic process shown below. Calculate the work done during the cycle. (Note: 1 MPa = 106 Pa.) Is the work positive or negative? Explain. What would the work be if the cycle were carried out in the counterclockwise direction?

|  | The area of a parallelogram is the base times the height. Thus W = (0.035 m3)(0.15 x 106 Pa) = + 5,250J Work would be negative going in the counterclockwise direction, changes in volume would be the negative of their clockwise values. |
| --- | --- |

10.	Calculate the net heat energy transfer into the system for the cycle in Question 9.

For the entire cycle, the change in internal energy, U, is zero.

Hence, U = Q - W = 0,   Q = W = 5,250J.

11.	Assuming that the process in Question 9 consists of two isobaric and two adiabatic processes, find the net heat energy input to the system. Hint: Use the equation for Q for isobaric processes on page 117, and recall the definition of an adiabatic process.

Heat is input into the system during the top leg of the cycle, characterized by P2 (=P3), V2 & V3.  One can use the formula relating heat, Q, to the change in temperature between points 2 and 3: Q = n Cp DT = n Cp (T3 - T2), where Cp is the specific heat at constant pressure for air, given in your lab write-ups as 29.0 J/mole-K.  To determine the temperature difference, use the ideal gas law:

PV = nRT;  T = PV/nR;  T3 - T2 = P3V3/nR - P2V2/nR = (P/nR) (V3-V2).  Substituting into above:

Q = nCp (T3 - T2)  = (nCp P/nR)  (V3-V2) = (Cp P/R)  (V3-V2) =

29.0 (J/mole-K) / 8.31 (J/mole-K)  0.25x106Pa  (.035m3)  =  30,500 J

12.	Find the efficiency of the cycle in Question 9.

e = W/Qin = 5,250 J / 30,500 J   =   0.17  or 17% efficiency.

13.	Find the efficiency of a Carnot engine operating between the highest and lowest temperatures in the cycle in Question 9.

The Carnot efficiency is given by: ec = 1 - Tc/Th.

Use the ideal gas law at points 1 and 3 to determine the temperature of the hot and cold reservoirs:

T1 = P11V1/nR;    T3 = P3V3/nR;  ec= 1 - T1/T3 = 1 - P1V1/ P3V3

1 -  (.10x106Pa x .03m3) / (.25x106Pa x .06m3)  = 1 - 0.2  =   0.8  or  80% efficiency.