SECTION L5: LAB 5—THE IDEAL GAS LAW

General Introduction

These investigations introduce students to the macroscopic quantities that characterize the state of a gas, pressure, volume and temperature. After an introduction to the measurement of pressure using a manometer and an electronic pressure sensor, students can examine the relationship between any pair of the quantities P, V and T. These can then be combined to give the Ideal Gas Law. Finally, using two simple simulations of gas particles in a cylinder, students can arrive at a simple, qualitative understanding of the microscopic basis for pressure in a gas.

Lab 5 Equipment and Supplies

•	2 translucent soda straws, about 20 cm long

•	length of Tygon tubing, about 20 cm long

•	small balloon

•	ring stand with two clamps to hold straws

•	computer-based laboratory system

• 	pressure sensor

•	RealTime Physics Heat and Thermodynamics experiment configuration files

• 	20-mL plastic syringe (with the needle removed)

• 	short piece of Tygon tubing (to attach syringe to pressure sensor)

• 	25-mL glass boiling flask with a one-hole stopper and T-connector

• 	large insulated container (e.g., Styrofoam cup)

• 	hot water (about 70°C)

•	ice

• 	temperature sensor

• 	ice

• 	Single Particle Movie (L05E3-1)

• 	Syringe Movie (L05E3-2)

• 	movie viewer software

Investigation 1: Measuring pressure

Equipment and Supplies

•	2 translucent soda straws, about 20 cm long

•	length of Tygon tubing, about 20 cm long

•	small balloon

•	ring stand with two clamps to hold straws

•	computer based laboratory system with one pressure sensor

• 	pressure sensor

•	RealTime Physics Heat and Thermodynamics experiment configuration files

Overview

Two methods for measuring pressure are briefly explored. First the principle behind a manometer is examined using a pair of straws and a small balloon. Then a calibrated, electronic pressure sensor is used to measure the pressure inside the balloon

Activity 1-1:  Measuring Pressure with a Manometer

The simply constructed manometer works well for the semi-quantitative observations in this activity. The balloon should just be blown up a little bit in order to avoid spilling water out of the straws.

Question 1-1:  The difference in height, y, will vary widely depending on how the balloon is blown up. Also, Patm depends on the altitude of the laboratory where the experiment is done. As a sample calculation with y = 10 cm and Patm = 1.013x105 Pa,

P = 1.013x105 + (1000)(9.80)(0.10) = 1.022x105 Pa

Activity 1-2:  Using a Computer-Based Pressure Sensor

Any computer-based pressure sensor should come with a calibration file.  More information on the pressure sensor needed for these activities is found in Section CS of this guide.

Question 1-2:  There was a significant difference in pressure. The pressure inside the balloon is atmospheric pressure plus the pressure caused by the stretched rubber of the balloon pushing on the gas inside.

Investigation 2: Behavior of a gas in terms of P, V and T

Equipment and Supplies

• 	20-mL plastic syringe (with the needle removed)

• 	short piece of Tygon tubing (to attach syringe to pressure sensor)

• 	computer-based laboratory system

• 	pressure sensor

• 	RealTime Physics Heat and Thermodynamics experiment configuration files

• 	25-mL glass boiling flask with a one-hole stopper and T-connector

• 	large insulated container (e.g., Styrofoam cup)

• 	hot water (about 70°C)

•	ice

• 	temperature sensor

• 	ice

Overview

As in Investigation 3 of Lab 4, the system examined is air in a syringe and/or a glass flask. To examine the relationship between any two of the quantities P, V and T, it is important to keep the third quantity constant. In the first activity the relationship between P and V is explored in an isothermal process, i.e., a compression in which the temperature of the gas is kept constant. This is accomplished by compressing the gas slowly so that it is always in thermal equilibrium with its surroundings.

Next the relationship between P and T is explored in an isovolumetric process, i.e., one in which the volume is kept constant. Finally, the relationship between V and T is explored in an isobaric process, i.e., one in which the pressure is kept constant.

These activities are more sophisticated than previous ones. Students must take some care in setting up the apparatus, carrying out the measurements and fitting the data.

Activity 2-1:  Isothermal Volume Change for a Gas

It is very important that students load the correct pressure sensor calibration file before taking any measurements. This is the first time that students will use prompted event mode in the software. This means that once the software is started, it will measure pressure continuously until a value is kept. Then, the volume may be entered manually.

It is important to include the volume of the syringe plus the volume of the tube and pressure sensor. You should give the students the volume of the pressure sensor. Also, either give them the inside diameter of the tubing, or have them measure it. Table 5-1 helps them to find the correct total volume to be entered into the computer for each experimental data point.

Figure L5-1 shows a graph for a typical experiment, with the experimental data fit with an inverse relationship.

| Figure L5-1: Pressure vs. volume graph for isothermal process for Activity 2-1. |  |
| --- | --- |

Question 2-1:  P and V appear to be inversely proportional.

Question 2-2:  PiVi = PfVf

Activity 2-2: Isovolumetric Temperature Change for a Gas

Since the volumes of the flask, tubing and pressure sensor do not change during the experiment, the processes in this activity are isovolumetric.  It is not important to know what the volume actually is.

Be sure that the students load the calibration files for the pressure and temperature sensor.

Both temperature and pressure are measured by electronic sensors.  With the software running in event mode, measurements are taken continuously by both sensors until the values are kept.  Then measurements are taken continuously again for the next data point.

Figure L5-2 shows a graph for a typical experiment with a linear fit to the data.

| Figure L5-2: Pressure vs. temperature graph for isovolumetric process for Activity 2-2. |  |
| --- | --- |

Question 2-3:  The relationship appears linear, and could be proportional since there is only a very small intercept on the pressure axis.

Question 2-4: Pi/Ti = Pf/Tf

Activity 2-3:  Isobaric Temperature Change for a Gas

In this activity, the total volume must be calculated from the volumes of the syringe, flask, tubing and sensor. Table 5-2 will help students to find the correct total volume to be entered into the computer for each experimental data point.

The software is run in prompted event mode as in Activity 2-3, so that the volume must be entered for each temperature that is kept.

Figure L5-3 shows a graph for a typical experiment with a linear fit to the data.

| Figure L5-3: Volume vs. temperature graph for isobaric process for Activity 2-3. |  |
| --- | --- |

Question 2-5:  The relationship is linear, and could be proportional since the intercept on the volume axis is relatively small.

Question 2-6: Vi/Ti = Vf/Tf

Question 2-7: According to the Ideal Gas Law, PV = (constant)T. If T is kept constant (isothermal process), then PV = constant, or P =constant/V, which is an inverse proportional relationship—Boyle’s law. If V is kept constant, then P = (different constant)T, which is a linear, proportional relationship—Gay-Lussac’s Law. Finally, if P is kept constant, then V = (different constant)T, which is a linear, proportional relationship—Charles’ Law.

Investigation 3:  Pressure and behavior of molecules in a gas

Equipment and Supplies

• 	Single Particle Movie (L05E3-1)

• 	Syringe Movie (L05E3-2)

• 	movie viewer software

Overview

In these extensions students will examine two simulations of gas molecules in a cylinder to help understand the origin of pressure on a microscopic basis, through a very simplified view of kinetic theory. The two movies are part of the RealTime Physics Heat and Thermodynamics experiment configuration files, and can be played on simple movie viewer software such as QuickTime Player or RealPlayer.

Question E3-1:  Chemicals come in definite proportions. There seem to have been a lot of chemical and physical experiments that confirm it.

Question E3-2:  The internal energy of the substance can increase. In the case of a gas, this means that the kinetic energy of random motions increases, and the molecules on the average move faster. This will make the pressure larger, and if the container can expand, the gas will expand to a larger volume.

Extension 3-1:  Origin of Pressure

Question E3-3:  The velocity component has the same magnitude but opposite direction after the collision. The magnitude of the change in momentum is 

px = 2mvx.

Question E3-4:  There must be a force since the molecule is accelerated, and its momentum changes. The force is the rate of change of momentum of the molecule. The molecule exerts an equal and opposite force on the wall.

Question E3-5:  There would be a larger force made up of the sum of forces exerted by all the molecules. Pressure is the force per unit area exerted by collisions of molecules with the walls of the container.

Extension 3-2:  Pressure vs. Volume

Question E3-6:  The number of collisions per unit time increases, and the momentum change of the particles also increases. The pressure should increase.

Question E3-7:  As V decreases, Boyle’s law says that pressure should increase.  This agrees with the answer to Question E3-6.

Question E3-8: As the temperature of the gas increases, the average speed of a molecule increases. Thus, molecules on the average have larger momentum, and, since they are moving faster, they make more collisions per unit time. Thus the pressure should increase as predicted by Gay-Lussac’s law.

Homework for Lab 5:  The Ideal Gas Law

1.	Explain the operation of a manometer. Why is the pressure related to the difference in heights of the two columns of water?

|  | The fluid at “a” experiences atmospheric pressure. At “b”, the pressure is atmospheric plus that caused by the “weight” of the fluid in the column above, P = Patm+mg/A = Patm+Vg/A = Patm+Ayg/A = Patm+gy. The pressure at “c” must match this, and the pressure at “d” will be the same. For the fluid to not move, the pressure at “e” must equal that at “d.” |
| --- | --- |

2.	Describe the evidence from this lab that supports Boyle's law, PV = constant for an isothermal process.  Describe any experimental difficulties you had in exploring this law.

The pressure and volume of air trapped in a syringe were recorded for several values of P and V.  All the while, the temperature of the air was assumed to be the same as room temperature and constant over time. By plotting the data points, it was determined that P and V were inversely proportional to one another. Thus the product of the two (PV) must be equal to a constant.

3.	Describe the evidence from this lab that supports Gay Lussac's law, P/T = constant for an isovolumetric process. Describe any experimental difficulties you had in exploring this law.

Using air at a constant volume (approx. 26 mL), the pressure of the air and the temperature of a bath in thermal equilibrium with the air were plotted and found to be proportional to one another (P  T).  Hence, the ratio of P over T must equal a constant.

4.	Describe the evidence from this lab that supports Charles' law, V/T = constant for an isobaric process. Describe any experimental difficulties you had in exploring this law.

An adjustable volume of air was kept in thermal equilibrium with a bath of known temperature.  Adjusting the volume of the system kept its pressure constant.  Thus volume could be compared to temperature at constant pressure. By graphing the data points, V and T were found to be proportional to one another, thus V/T is constant.

5.	Which of the three laws was the most difficult to explore experimentally?  Why?

The experiment to verify Charles’ law gave the least satisfactory data. For this experiment, a large portion of the air in the system was NOT in contact with the bath, thus temperature could vary substantially across the system.

6.	Why was it necessary to keep one of the variables P, V or T constant in each activity?

By holding one of the variables constant, the relationship of the other two could be assessed. By holding each of the variables constant in turn, the relationship between the three sets of the other two variables could be determined, and a relationship between all three (the Ideal Gas Law:

PV = nRT) could be verified. If one didn’t hold one of the variables constant, then the relationship between one and each of the others would come into play, with unknown consequences.

7.	The Ideal Gas Law is given as PV = nRT, where n is the number of moles of gas (1 mole = 6.02 x 1023 molecules) and R is the universal gas constant (R = 8.31 J/mole-K). Explain why the product of pressure and volume is constant in an isothermal process.

In an isothermal process, the temperature is kept constant. Thus PV is the product of three constants (provided the quantity of gas, n, is kept constant), and must be constant.

Explain why the ratio of pressure and temperature is constant in an isovolumetric process.

In an isovolumetric process, the volume is kept constant. Thus P= (nR/V)T, or P =(constant)T and P/T = constant.

8.	Explain the origin of pressure of a gas in a container based on the motions of molecules in the gas, their average momentum and their collisions with the walls of the container.

For a molecule (or atom) of the gas with mass m, with an average velocity vx, an elastic collision with the container wall effects a change in momentum of magnitude 2mvx. The relationship between the impulse exerted by the wall to stop and reverse the molecule’s direction, and the change in momentum tells us that:  F t = p,  F = p/t,  the rate of momentum change for the molecule is the same as the force exerted by the wall on the molecule. This force, exerted over the area of the wall matches the pressure exerted by molecules on the wall.

9.	Based on the kinetic theory definition of temperature as the average kinetic energy of a molecule in an ideal gas and your answer to Question 8, explain why the pressure of a gas increases when it is compressed while kept at a constant temperature.

For an ideal gas, the temperature is proportional to the average kinetic energy of the molecules that in turn, is proportional to the average of the square the molecule’s velocity:

3/2 k T = K.E.av = (1/2 m v2)av

Thus if the temperature remains the same, so does the average kinetic energy of a molecule of the gas.  The number of collisions with the container walls within a given time interval, however, increases as the walls of the container are moved closer together (compression).  Hence the pressure of the gas increases.

10.	Based on the kinetic theory definition of temperature as the average kinetic energy of a molecule in an ideal gas and your answer to Question 8, explain why the pressure of a gas increases when its temperature is increased while its volume is kept constant.

Following the answer to (8), if the temperature increases, a molecule’s average velocity increases.  The rate of momentum change (2m|v|) increases, and so must the average force on the molecule exerted by the wall. Therefore the pressure exerted by the gas on the wall, which equals this force divided by the area of the wall, increases.