SECTION L10:  LAB 10—Electromagnetism

General Introduction

The investigations in Lab 10: Electromagnetism guide students to study the relationships between electricity and magnetism. In Investigation 1, the magnetic field of a current-carrying wire is examined qualitatively and quantitatively. In Investigation 2, electromagnetic induction by a magnet in a coil of wire is examined, and Faraday’s law is introduced. In Investigation 3, electromagnetic induction is explored quantitatively using video analysis. Finally, Lenz’s law is studied in Investigation 4.

Lab 10 Equipment and Supplies List

• 	2 6 V batteries

• 	3 30 cm lengths of wire with alligator clip leads

• 	switch

• 	5 ohm resistor

• 	non-magnetic (aluminum) ring stand w/ clamps and rods

• 	20 cm x 20 cm piece of cardboard (or acrylic plate) with a hole

• 	small compass

• 	computer-based laboratory system

• 	RealTime Physics Electricity and Magnetism experiment configuration files

• 	magnetic field sensor

• 	strong rod-shaped magnet

• 	coil of wire with large number of turns

• 	an analog galvanometer, voltmeter or ammeter

• 	something to hold the magnet in place, such as a lab stand

• 	a coil of wire with ends exposed so that the direction of the winding can be determined

Investigation 1:  magnetism from electricity

Equipment and Supplies

• 	2 6 V batteries

• 	3 30 cm lengths of wire with alligator clip leads

• 	switch

• 	5 ohm resistor

• 	non-magnetic (aluminum) ring stand w/ clamps and rods

• 	20 cm x 20 cm piece of cardboard (or acrylic plate) with a hole

• 	small compass

• 	computer-based laboratory system

• 	RealTime Physics Electricity and Magnetism experiment configuration files

• 	magnetic field sensor

Overview

The magnetic field produced by an electric current is explored qualitatively and quantitatively using a compass and a magnetic field sensor.

Activity 1-1: Magnetic Field of a Straight Current-Carrying Wire

Question 1-1: The magnetic field lines are circles with the wire through their centers. With the current flowing up through the wire, the magnetic field circulates in a counterclockwise direction as viewed from above.

Question 1-2: The field lines are the same, but the field circulates in a clockwise direction as viewed from above.

Question 1-3: With the wire grasped by the right hand, with the thumb pointing in the direction of current flow, the magnetic field lines are circles around the wire in the direction that the fingers circulate around the wire.

| Question 1-4: The field magnitude gets smaller as you move radially away from the wire. | SS S W WW |
| --- | --- |

Question 1-5: The magnitude of the field varies in the same way on both sides of the wire.

Question 1-6: The field magnitude decreases as the current decreases.

Question 1-7: The magnitude of the magnetic field decreases with distance from the wire, and is proportional to the current flowing through the wire. The field lines are circles with the wire through their center. The righthand rule tells in which direction the magnetic field circulates around the wire.

Investigation 2:  interactions between a Magnet and a coil

Equipment and Supplies

• 	strong rod-shaped magnet

• 	coil of wire with large number of turns

• 	an analog galvanometer, voltmeter or ammeter

• 	something to hold the magnet in place, such as a lab stand

Overview

In this investigation, the voltages (emfs) induced in a coil of wire by its interactions with a permanent magnet are examined.

Activity 2-1: Magnet and Coil

Question 2-1: There was no electrical effect when the magnet remained at rest in the center of the coil. When the magnet was moved in or out from the center of the coil, there was a voltage observed with the meter.

Question 2-2: The voltage moving the magnet in was the opposite sign of the voltage moving he magnet out.

Question 2-3: The signs of the voltages observed are reversed when the poles of the magnet are reversed.

Activity 2-2: Coil Moving Near a Magnet

Question 2-4: Voltages were observed when the coil was moved away from and moves towards the magnet.

Question 2-5: Moving towards and moving away give the opposite sign of voltage.

Question 2-6: Reversing the poles reverses the sign of the voltages observed.

Question 2-7: Moving the coil away from the North pole of the magnet resulted in the same effect as moving the North pole of the magnet away from the coil. Moving the coil away toward the North pole of the magnet resulted in the same effect as moving the North pole of the magnet toward the coil. Likewise for the South pole of the magnet.

Question 2-8: The same relative motion between the magnet pole and the coil seems to give the same observed voltage.

Activity 2-3: The Magnetic Field of a Rod-shaped Magnet

Question 2-9: Magnetic field in the vicinity of the coil decreases in magnitude in both cases.

Question 2-10: Voltages are only observed when the magnet or coil is moving, i.e., when the field in the vicinity of the coil is changing.

Question 2-11: When the magnet is just sitting in the center of the coil, the magnetic field in the vicinity of the coil is not changing, so there is no voltage. This is consistent with Question 2-10.

Question 2-12: You can change the magnetic flux by (1) changing the magnitude of the field, (2) changing the angle between the field and the face of the coil, or (3) changing the area of the coil.

Question 2-13: In all of the observations, when the magnet is moved relative to the coil, or the coil relative to the magnet, you are (1) changing the magnitude of the field.

Activity 2-4: Dependence of Induced EMF on Speed of Moving Magnet

Question 2-14: Pulling the magnet out more quickly produced a larger voltage.

Question 2-15: The change in flux is the same whether the magnet is pulled out slowly or quickly.

Question 2-16: Since the induced emf is larger when the magnet is pulled out faster, the induced emf must depend on the rate of change in magnetic flux, not just the change in magnetic flux.

Extension 2-5: How Does Induced EMF Depend on the Speed of a Coil’s Rotation?

Question E2-17: There was an induced emf when the coil was rotated. The changing magnetic flux came about by (2) changing the angle between the field and the face of the coil.

Question E2-18: The faster the coil was rotated, the larger was the induced emf.

Question E2-19: The change in flux is the same whether the coil is rotated slowly or quickly.

Question E2-20: Since the induced emf is larger when the coil is rotated faster, the induced emf must depend on the rate of change in magnetic flux, not just the change in magnetic flux.

Question E2-21: In all of these observations where there was an induced emf, there was a changing flux. The faster the change in flux, the larger the induced emf. These observations are completely consistent with Faraday’s law.

Investigation 3: FARADAY’S LAW AND ELECTROMAGNETIC INDUCTION—A MORE QUANTITATIVE LOOK

Equipment and Supplies

• 	RealTime Physics Electricity and Magnetism experiment configuration files

Overview

In this Investigation students will look at videos of various motions of a permanent magnet through a coil, and try to figure out what the voltage (emf) induced in the coil should look like.

Question 3-1: Answer depends on student work. They should be similar.

Activity 3-1: Practice Observing Data for Induced EMF

Question 3-2: Answer depends on student prediction. The magnet moves slowly into the coil, and then quickly out of the coil. The absolute value of the emf is smaller as the magnet moves in than when it moves out.

Activity 3-2: Analyzing Different Situations

Question 3-3: Graph 1 corresponds to Movie B. In Movie B, the North pole moves into the coil quickly and then out slowly. From the Sample Movie, North moving in produced a positive emf. Therefore, the motion in Movie B should give a large positive emf followed by a smaller negative emf. Only Graph 1 has these features.

Question 3-4: Graph 2 corresponds to Movie D. In Movie D, the South pole moves into the coil quickly and then out quickly. From the Sample Movie, North moving in produced a positive emf, so South pole moving in should produce a negative emf. Therefore, the motion in Movie D should give a large negative emf followed by a large positive emf. Only Graph 2 has these features.

Question 3-5: Graph 3 corresponds to Movie A. In Movie A, the South pole moves into the coil slowly and then out quickly. From the Sample Movie, North moving in produced a positive emf, so South pole moving in should produce a negative emf. Therefore, the motion in Movie A should give a small negative emf followed by a larger positive emf. Only Graph 3 has these features.

Question 3-6: Graph 4 corresponds to Movie C. In Movie C, the North pole moves into the coil slowly and then out slowly. From the Sample Movie, North moving in produced a positive emf. Therefore, the motion in Movie C should give a small positive emf followed by a small negative emf. Only Graph 4 has these features.

Investigation 4: LENZ’S LAW AND ELECTROMAGNETIC

INDUCTION

Equipment and Supplies

• 	a coil of wire with ends exposed so that the direction of the winding can be determined

• 	a strong rod-shaped magnet

• 	an analog galvanometer, ammeter or voltmeter

Overview

In this Investigation the direction (or sense) of the induced emf is examined, and interpreted in terms of Lenz’s law. In order to do this, it is necessary to keep careful track of the direction of the emf.

Activity 4-1: Lenz’s Law

Question 4-1: The induced current is positive, i.e., it must flow clockwise around the coil as viewed from the right, and into the + terminal of the meter.

Question 4-2: The current induced in the coil produces a magnetic field that points toward the left. (Rotating your fingers around the outside of the coil in a clockwise direction (as viewed from the right), your thumb points to the left.

Question 4-3: Moving the North pole of the magnet into the coil increases the magnetic flux toward the right through the coil.

Question 4-4: The flux produced by the induced current in the coil is toward the left, as explained in Question 4-2. There, it subtracts from the increase in flux through the coil produced by the movement of the magnet.

Question 4-5: The induced current is caused by the increase in flux through the coil to the right caused by the movement of the North pole closer to the left end of the coil. Therefore, the field produced by the induced current opposes the effect that is producing it—it opposes this increase in flux.

Extension 4-2: Another Test of Lenz’s Law

Question E4-6: The current is again positive, i.e., it flows around the coil clockwise as viewed from the right. This time, the flux toward the left produced by the South pole of the magnet is decreased as the pole is pulled out from the center of the coil. Since the induced current is again positive, it again produces an induced field and flux to the left, that tries to compensate for the decrease in the flux to the left as the South pole is pulled out from the coil.

homework for Lab 10: ELECTROMagnetism

| 1. In the diagram on the right, a wire is carrying a current. The magnetic field above the wire is into the paper, and the magnetic field below the wire is out of the paper, as shown. (An x indicates a field line into the paper and a • indicates a field line out of the paper.) What is the direction of current flow in the wire? Explain your answer based on your observations in this lab and the right hand rule. |  |
| --- | --- |

The direction of current flow in the wire is toward the left. If you grasp the wire with your thumb pointing to the left, your fingers circulate around the wire in the direction of the indicated field lines. Therefore, the current is toward the left.

2. 	Is the magnetic field magnitude larger at point 1 or point 2? Explain based on your observations in this lab.

The magnetic field is stronger at point 1 than at point 2. This was observed in Investigation 1 using he magnetic field sensor.

3. 	You have a rod-shaped magnet and also a coil of wire connected to a voltmeter. Under which of the following circumstances will there be a voltage induced across the coil? If you predict that a voltage is induced, explain why. If you think there will be no voltage, explain why not.

a. 	The magnet is sitting near the coil, and the coil is rotated so that its face is no longer perpendicular to the magnet.

A voltage will be induced. The flux through the coil is changed because the angle between the direction of the field and the normal to the face of the coil is changed.

b. 	The magnet is sitting near the coil, and the coil is moved quickly away from the magnet.

A voltage will be induced. The flux through the coil is changed because the magnetic field in the vicinity of the face of the coil is changed.

c. 	The magnet is sitting at rest with its North pole in the middle of the coil.

No voltage will be induced. The flux through the coil is unchanged.

d. 	The magnet is sitting with its North pole in the middle of the coil and is pulled away from the coil.

A voltage will be induced. The flux through the coil is changed because the magnetic field in the vicinity of the face of the coil is changed.

4. 	If there is a changing flux through the coil in any of the circumstances described in (3), explain what is causing the flux to change.

a.	The angle between the normal to the coil and the direction of the magnetic field is changing.

b.	The magnetic field in the vicinity of the face of the coil is changed.

d.	The magnetic field in the vicinity of the face of the coil is changed.

5. 	Calculate the magnetic flux through the coil in each case. The magnetic field is 2.0 Tesla, and the area of the face of the coil is 0.25 m2.

F=BDAcosq=(2.0)(0.25)(1)=0.50Tm2			F=BDAcosq=(2.0)(0.25)(cos30°)=0.43Tm2	F=BDAcosq=(2.0)(0.25)(cos90°)=0

6. 	The North pole of a rod-shaped magnet is near the center of a coil of wire connected to an ammeter as shown below. The coil is pulled quickly away from the magnet. Use Lenz’s law to predict whether the reading on the meter will be positive or negative. Explain your answer carefully.

The field in the vicinity of the face of the coil is to the right. As the coil is pulled away, the flux toward the right is decreasing. Therefore, the current induced in the coil must flow so as to produce a magnetic field and flux toward the right. Using the righthand rule, the induced current must flow around the coil in a counterclockwise direction as viewed from the right. Thus it flows out from the + terminal of the meter, and is gives a negative current reading.