SECTION IN:  INTRODUCTION

Development of the series of RealTime Physics (RTP) laboratory guides began in 1992 as part of an ongoing effort to create high quality curricular materials, computer tools, and apparatus for introductory physics teaching.1 The RTP series is part of the Activity-Based Physics Suite2 of curricular materials that include the Tools for Scientific Thinking laboratory modules3, the Workshop Physics Activity Guide4,5, and the Interactive Lecture Demonstration series6,7. The development of all of these curricular materials has been guided by the outcomes of physics education research.  This research has led us to believe that students can learn vital physics concepts and investigative skills more effectively through guided activities that are enhanced by the use of powerful microcomputer-based laboratory (MBL) tools.

The use of MBL tools and video analysis for both conceptual and quantitative activities, when coupled with physics education research, has led us to expand our view of how the introductory physics laboratory can be redesigned to help students learn physics more effectively.

Since 1986, new MBL tools—originally developed at Technical Education Research Centers (TERC) and at the Center for Science and Mathematics Teaching, Tufts University—have become increasingly popular for the real-time collection, display, and analysis of data in the introductory laboratory.  MBL tools consist of electronic sensors, a microcomputer interface, and software for data collection and analysis.  Sensors are now available for force, sound, magnetic field, current, voltage, temperature, pressure, rotary motion, acceleration, humidity, light intensity, pH, dissolved oxygen, and others.

MBL tools provide a powerful way for students to learn physics concepts. For example, students who walk in front of an ultrasonic motion sensor while the software displays position, velocity and/or acceleration vs. time in real time more easily discover and understand motion concepts.  They can see a cooling curve displayed instantly when a temperature sensor is plunged into ice water, or they can sing into a microphone and see a pressure vs. time plot of sound intensity.

MBL data can also be analyzed quantitatively. Students can obtain basic statistics for all or a selected subset of the collected data, and then either fit or model the data with an analytic function. They can also integrate, differentiate, or display a fast Fourier transform of data.  Software features enable students to generate and display calculated quantities from collected data in real time. For example, since mechanical energy depends on mass, position, and velocity, the time variation of potential and kinetic energy of an object can be displayed graphically in real time. The user just needs to enter the mass of the object and the appropriate energy equations ahead of time.

Short videos, often just 20 to 30 frames in length, can be extremely useful in teaching physics and other sciences. Not still photographs, but too short to be considered movies, these "live photos" are designed for analysis using a computer. Positions of objects in the video frame can be measured by pointing a mouse and clicking. The data can be graphed, analyzed in spreadsheets, compared to theoretical models, and even used to display vectors or points superimposed on the original video. The LivePhoto Physics Project has created a collection of videos for use in activities in many areas of physics.8

COMMON ELEMENTS IN THE  REALTIME PHYSICS SERIES

Each laboratory guide includes activities for use in a series of related laboratory sessions that span an entire quarter or semester. Lab activities and homework assignments are integrated so that they depend on learning that has occurred during the previous lab session and also prepare students for activities in the next session. The major goals of the RealTime Physics project are: (1) to help students acquire an understanding of a set of related physics concepts; (2) to provide students with direct experience of the physical world by using MBL tools for real-time data collection, display and analysis, (3) to enhance traditional laboratory skills; and (4) to reinforce topics covered in lectures and readings using a combination of conceptual activities and quantitative experiments.

To achieve these goals we have used the following design principles for each module based on educational research.

• 	The materials for the weekly laboratory sessions are sequenced to provide students with a coherent observational basis for understanding a single topic area in one semester or quarter of laboratory sessions;

• 	The laboratory activities invite students to construct their own models of physical phenomena based on observations and experiments;

• 	The activities are designed to help students modify common pre-conceptions about physical phenomena that make it difficult for them to understand essential physics principles;

• 	The activities are designed to work best when performed in collaborative groups of 2 to 4 students;

• 	MBL tools and video analysis are used by students to collect and graph data in real time so they can test their predictions immediately;

• 	A learning cycle is incorporated into each set of related activities that consists of prediction, observation, comparison, analysis, and quantitative experimentation;

• 	Opportunities are provided for class discussion of student ideas and findings;

• 	Each laboratory comes with a pre-lab warm-up assignment, and with a post-lab homework assignment that reinforces critical physics concepts and investigative skills.

The core activities for each laboratory session are designed to be completed in two hours. Extensions have been developed to provide more in-depth coverage when longer lab periods are available. The materials in each laboratory guide are comprehensive enough that students can use them effectively even in settings where instructors and teaching assistants have minimal experience with the curricular materials.

Instructors can make local modifications to adapt the RealTime Physics labs to local needs, and have custom versions printed through Wiley’s Custom Select custom publishing. Activity-Based Physics curricular materials can be combined in various ways to meet the needs of students and instructors in different learning environments. A helpful guide in planning use of these materials in conjunction with other Suite materials is Teaching Physics with the Physics Suite by Redish2

The RealTime Physics laboratory guides are designed as the basis for a complete introductory physics laboratory program at colleges and universities. But, they can also be used as the central component of a high school physics course.  In a setting where formal lectures are given we recommend that the RTP laboratories be used in conjunction with Interactive Lecture Demonstrations.6,7

THE ELECTRICITY AND MAGNETISM LABORATORY GUIDE

The primary goal of this RealTime Physics Electricity and Magnetism laboratory guide is to help students achieve a solid understanding of basic electricity and magnetism concepts and DC circuits, including Kirchhoff’s circuit rules. A number of physics education researchers have documented that most students begin their study of electric circuits with serious misconceptions.9,10 The majority of students in the introductory course have had little or no experience working with circuits, and they commonly confuse even the meanings of such basic circuit quantities as current, voltage, potential and potential difference. For many such students, even the requirement of a complete circuit for current to flow is foreign.

RealTime Physics Electricity and Magnetism includes 10 labs covering basic electricity and magnetism concepts and DC circuits:

Lab 1 (Electric Charges, Forces and Fields) This lab introduces students to the electrostatic interactions, Coulomb’s law and the concept of an electric field. Electrostatic forces are first explored using charged pieces of Scotch© tape, to explore how many types of electric charge there are, and how they interact. Observations with foam cups charged in different ways introduce the concepts of conductors and non-conductors, and charging by induction. Coulomb’s law is explored quantitatively by analyzing a video of a charged ball hanging from a string interacting with a charged prod. Finally, the concept of electric field is introduced by examining the force on a piece of charged tape in the vicinity of a charged rod.

Lab 2 (Electric Fields, Flux and Gauss’ Law) This lab continues the study of electric field, first by examining Gauss’s law in two dimensions. Then the concept of electric flux is introduced, and its dependence on the angle between the field lines and the surface area is examined quantitatively using a model fabricated from nails and a wire ring. Finally, the distribution of excess charge on a conductor is analyzed using a variation of Faraday’s ice pail experiment.

Lab 3 (Electric and Gravitational Potential) The gravitational and electrostatic forces are first compared. Then, the independence of path for gravitational work is explored quantitatively using a force sensor to measure the force applied to an object to move it along various paths. Equipotential “surfaces” in two dimensions as explored using electrodes painted on special conducting paper, and field lines are constructed using the equipotentials.

Lab 4 (Batteries, Bulbs and Current): This lab introduces students to the concept of electric current as a flow of electrical charge. Students experiment with connections of very simple circuits to establish the need for a complete circuit. They explore a model for how current flows in a closed circuit using current sensors, and establish that the current is the same everywhere in a series circuit. They also examine batteries as constant voltage sources (rather than constant current sources) using a current and a voltage sensor. The have practice in drawing circuit diagrams and in designing simple circuits using basic circuit elements. Finally they examine a mechanical analogue for current flow and resistance..

Labs 5 and 6 (Current in Simple DC Circuits and Voltage In Simple DC Circuits and Ohm’s Law): In these two labs, students use current and voltage sensors to examine the current and voltage relationships in simple series and parallel circuits. They examine increasingly sophisticated circuits, and also explore the effects of the internal resistance of a battery—refining their ideas about a battery being a constant voltage source. Finally, they examine Ohm’s law quantitatively.

Lab 7 (Kirchhoff’s Circuit Rules): In this lab, the use of a multi-meter, and its connection as an ammeter, voltmeter and ohmmeter are first explored. The laws for connection of resistors in series and parallel are discovered. Finally, more sophisticated circuits are analyzed using Kirchhoff’s rules.

Lab 8 (Introduction to Capacitors and RC Circuits): This lab first explores the meaning of capacitance, and its dependence for a parallel plate capacitor on the area and separation of the plates. Next, the laws for addition of capacitors in series and in parallel are discovered. Finally, RC circuits are examined qualitatively using current and voltage sensors to display graphs for charging and discharging a capacitor through a resistor. Ceramic magnets are used to explore the inseparability of N and S magnetic poles.

Lab 9 (Magnetism) Students first examine the interactions of permanent magnets with various objects, and establish the existence of two types of magnetic poles. Then the orientation of hanging magnets in the Earth’s magnetic field is observed. The magnetic field around a bar magnet is explored qualitatively with compasses and iron filings, and quantitatively with a magnetic field sensor. The behavior of magnetic materials is examined by magnetizing paper clips using a permanent magnet. Finally, the magnetic forces on a moving charge and on a current-carrying wire are examined, and the Lorentz force law and right-hand rule are introduced.

Labs 10 (Electromagnetism) The magnetic field produced by a current-carrying wire is examined semi-quantitatively using a compass and a magnetic field sensor. Then production of an emf through the relative motion of a permanent magnet and a coil of wire is explored. Magnetic flux and Faraday’s law are introduced. Then students explore semi-quantitatively the relationship between the induced emf and the motion of a permanent magnet near a coil by analyzing videos of this motion, and graphs of the induced emfs. Finally, Lenz’s law is introduced and explored by examining some electromagnetic induction experiments more carefully.

ON-LINE TEACHERS’ GUIDE

This Teachers’ Guide focuses on pedagogical (teaching and learning) aspects of using the curriculum, as well as computer-based and other equipment.  The Guide is offered as an aid to busy physics educators and does not pretend to delineate the "right" way to use the RealTime Physics Electricity and Magnetism curriculum and certainly not the MBL tools.  There are many right ways.   The Guide does, however, explain the educational philosophy that influenced the design of the curriculum and tools and suggests effective teaching methods.  Most of the suggestions have come from the college, university and high school teachers who have participated in field-testing of the curriculum.

This Guide has thirteen sections besides this introduction.  Section II presents suggestions regarding computer hardware and software to aid in the implementation of this activity-based MBL curriculum.  Sections III through XII present information about the ten different laboratories.  Included in each of these is information about the specific equipment and materials needed, tips on how to optimize student learning, answers to questions in the labs and complete answers to the homework.

EXPERIMENT CONFIGURATION FILES

Experiment configuration files are used to set up the appropriate software features to go with the activities in these labs.  You will need the set of files that is designed for the software package you are using, or you will need to set up the files yourself.  Experiment configuration files for RealTime Physics Electricity and Magnetism are available for the Vernier software package, Logger Pro (for Windows and Macintosh), and for the PASCO Data Studio (for Macintosh and Windows). Appendix A of this guide outlines the features of the experiment configuration files for RealTime Physics Electricity and Magnetism.

CONCLUSIONS

RealTime Physics Electricity and Magnetism has been used in a variety of different educational settings. Many university, college and high school faculty who have used this curriculum have reported improvements in student understanding of electricity, magnetism and electric circuits. For circuits, these comments are supported by our careful analysis of pre-and post-test data using the Electric Circuits Conceptual Evaluation, some of which have been reported in the literature.11  Similar research on the effectiveness of RealTime Physics Mechanics 12,13,14 and Heat and Thermodynamics, also shows dramatic conceptual learning gains in these topic areas. We feel that by combining the outcomes of physics educational research with microcomputer-based tools, the laboratory can be a place where students acquire both a mastery of difficult physics concepts and vital laboratory skills.

ACKNOWLEDGMENTS

RealTime Physics Electricity and Magnetism could not have  been developed without the hardware and software development work of Stephen Beardslee, Lars Travers, Ronald Budworth and David Vernier.  We are indebted to numerous college, university and high school physics teachers, and especially Curtis Hieggelke (Joliet Junior College), John Garrett (Sheldon High School) and Maxine Willis (Gettysburg High School) for beta testing earlier versions of the laboratories with their students. At the University of Oregon, we especially thank Dean Livelybrooks and Stan Micklavzina for supervising the introductory physics laboratory, for providing invaluable feedback and for writing some of the homework solutions for the Teachers’ Guide.  Frank Womack, Dan DePonte and all of the introductory physics laboratory teaching assistants provided valuable assistance and input.  We also thank the faculty at Tufts University and Dickinson College for their suggestions in developing RealTime Physics Electricity and Magnetism, and for assisting with our conceptual learning assessments. Finally, we could not have even started this project if not for our students’ active participation in these endeavors.

This work was supported in part by the National Science Foundation under grant number DUE-9455561, “Activity Based Physics:  Curricula, Computer Tools, and Apparatus for Introductory Physics Courses,”  grant number USE-9150589,  "Student Oriented Science," grant number DUE-9451287, “RealTime Physics II: Active University Laboratories Based on Workshop Physics and Tools for Scientific Thinking,” grant number USE-9153725, "The Workshop Physics Laboratory Featuring Tools for Scientific Thinking,"  and grant number TPE-8751481, "Tools for Scientific Thinking:  MBL for Teaching Science Teachers,"  and by the Fund for Improvement of Post-secondary Education (FIPSE) of the U.S. Department of Education under grant number  G008642149, "Tools for Scientific Thinking," and number P116B90692, "Interactive Physics."

REFERENCES

1David R. Sokoloff, Ronald K. Thornton and Priscilla W. Laws, “RealTime Physics: Active Learning Labs Transforming the Introductory Laboratory,” Eur. J. of Phys., 28 (2007), S83-S94.

2Edward F. Redish, Teaching Physics with the Physics Suite (Hoboken, NJ, Wiley, 2003).

3Ronald K. Thornton  and David R. Sokoloff, "Tools for Scientific Thinking--Heat and Temperature Curriculum and Teachers' Guide," (Portland, Vernier Software, 1993) and David R. Sokoloff and Ronald K. Thornton, "Tools for Scientific Thinking--Motion and Force Curriculum and Teachers' Guide," Second edition, (Portland, Vernier Software, 1992).

4P. W. Laws, "Calculus-based Physics Without Lectures," Phys. Today 44: 12, 24-31 (December, 1991).

5Priscilla W. Laws, Workshop Physics Activity Guide:  The Core Volume with Module 1: Mechanics, (Wiley, New York, 1997).

6David R. Sokoloff and Ronald K. Thornton, Interactive Lecture Demonstrations (Hoboken, NJ, Wiley, 2004).

7David R. Sokoloff and Ronald K. Thornton, “Using Interactive Lecture Demonstrations to Create an Active Learning Environment,” The Physics Teacher 27: 6, 340 (1997).

8 See http://livephoto.rit.edu/ and Priscilla Laws, Robert Teese, Maxine Willis and

Patrick Cooney, Physics with Video Analysis (Beaverton, OR, Vernier Software and Technology, 2009).

9L. C. McDermott, "Millikan lecture 1990: What We Teach and What is Learned--Closing the Gap," Am. J. Phys 59, 301 (1991).

10Lillian C. McDermott and Peter S. Shaffer, “Research as a Guide to Curriculul Development:  An Example from Introductopry Electricity.  Part I:  Investigation of Student Understanding,” American Journal of Physics 60(11), 1992, pp. 994-1003.

11David R. Sokoloff, "Teaching Electric Circuit Concepts Using Microcomputer-Based Current and Voltage Probes," chapter in Microcomputer-Based Labs:  Educational Research and Standards, Robert F. Tinker, ed., Series F, Computer and Systems Sciences,  156, 129-146 (Berlin, Heidelberg, Springer Verlag, 1996).

12Ronald K. Thornton and David R. Sokoloff, "Assessing Student Learning of Newton's Laws:  The Force and Motion Conceptual Evaluation and the Evaluation of Active Learning Laboratory and Lecture Curricula," Am. J. Phys.  64 , 338-352 (1998).

13Ronald K. Thornton, "Learning Physics Concepts in the Introductory Course:  Microcomputer-based Labs and Interactive Lecture Demonstrations," in Conference on the Introductory Physics Course,  J.W. Wilson, ed. (Wiley, New York, 1997), p 69-85.

14Ronald K. Thornton and David R. Sokoloff, "Learning Motion Concepts Using Real-Time Microcomputer-Based Laboratory Tools," Am. J.. Phys. 58, 858 (1990).

|  | This project was supported, in part, by the Fund for Improvement of Post-Secondary Education (FIPSE) and the National Science Foundation. Opinions expressed are those of the authors and not necessarily those of the foundations. |
| --- | --- |