SECTION L5:  LAB 5—CURRENT IN SIMPLE DC CIRCUITS

General Introduction

In this laboratory students investigate current flow in parallel and series connections. The investigation leads students to refine their model for current flow developed in the previous lab. It also gives students an opportunity to discover the current sourcing characteristics of a battery.

In Investigation 1 students explore the relationship between current and number of bulbs in a series circuit. The students observe qualitative data about the currents from bulb brightness and they observe quantitative data using the current sensors.  Students learn that the current through a branch decreases as the number of bulbs connected in series in the branch increases.

In Investigation 2, students use the same qualitative and quantitative methods to investigate characteristics of current flow through parallel circuits. Bulb brightness and current sensor data are collected in exercises with a number of parallel circuits.

In Investigation 3 students combine the knowledge they gained in Investigations 1 and 2 to analyze and investigate more complex circuits that contain both series and parallel connections. They also develop further their understanding of parallel and series connections through circuit diagramming exercises.

Lab 5 Equipment and Supplies List

•	computer-based laboratory system

• 	two current sensors

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	a very fresh, alkaline 1.5-V D cell battery with holder

•	8 wires with alligator clip leads

•	4  #14 bulbs in sockets

•	contact switch

•	two 10-ohm resistors

Investigation 1:  Current Flow in Series circuits

Equipment and Supplies

•	computer-based laboratory system

• 	two current sensors

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	a very fresh, alkaline 1.5-V D cell battery with holder

•	6 wires with alligator clip leads

•	4  #14 bulbs in sockets

•	contact switch

•	two 10-ohm resistors

Overview

In this investigation students explore the current flow in a series circuit with different numbers of bulbs. Students first observe qualitatively the differences in currents through circuits with one and two bulbs. Bulb brightness provides qualitative data. Students then use current sensors to quantitatively investigate whether current is ‘used up’ as it flows through a circuit.

Activity 1-1:  The Relative Brightness of Bulbs

It is important that the batteries are fresh.

Question 1-1: Bulb A is brighter than bulbs B and C.  Bulbs B and C have the same brightness

Question 1-2: There is no change in the brightness: bulbs B and C are the same brightness, and bulb A is brighter than B and C

Question 1-3: The brightness of the bulb gives no information about the direction of the current through the circuit; the brightness does not change when the bulb is hooked up to the opposite terminals of the battery.

Activity 1-2: Current in a Simple Circuit with Bulbs

If the bulbs were ideal (linear) resistors, the current for the circuit with two bulbs would be exactly half the value of the current for the circuit with one bulb. The bulbs however have a nonlinear resistance. Therefore the current observed for the circuit with two bulbs will be more than half the value of the current for the circuit with one bulb. The objective of this activity is to have students confront the common misconception that current is ‘used up’ in a circuit. In this activity students discover that current is not  ‘used up.’ Because of the nonlinearity of the bulbs, we cannot investigate the linear Ohm’s Law relation.

It is important to zero the current sensors before measurements begin.

| Figure L5-1: Current vs. Time for current entering and exiting one bulb in the battery and bulb circuit graphed for Activity 1-2 and Figure 5-2(a). |  |
| --- | --- |

| Figure L5-2: Graph of Current vs. Time for current entering 2 different bulbs in the series battery and 2-bulb circuit graphed for Activity 1-2 and Figure 5-2(b). |  |
| --- | --- |

Question 1-4:  The currents into and out of bulb A are equal.  (Small differences will be due to differences in calibration—students may wish to switch sensors with each other to confirm.)

Question 1-5: Current is the same in both bulbs.

Question 1-6: The current in bulb A is greater than the current through bulbs B and C. The currents through bulbs B and C are equal to each other.

Question 1-7:  The bulb is brighter when more current flows through it.

Question 1-8:  The current delivered by the battery is greater for the single bulb circuit than for the double bulb circuit. The addition of another bulb decreases the current through the original bulb.

Question 1-9:  If the bulbs provide a resistance to the flow of current, more bulbs in the circuit would provide more resistance to current flow, thus the current would be lower.

Question 1-10:  Current decreases as the total resistance of the circuit increases, and vice-versa.

Activity 1-3: Current in a Simple Circuit with Resistors

Question 1-11: The current in resistor A is greater than the current through resistors B and C. The currents through resistors B and C are equal to each other.

Question 1-12: The current delivered by the battery is greater for the single resistor circuit than for the double resistor circuit. The addition of another resistor decreases the current through the original resistor.

Question 1-13: The current in the circuit with two resistors in series is exactly half that in the circuit with just one resistor.

Question 1-14: The current in a series circuit is reduced by a factor equal to the number of identical resistors connected in series.

Question 1-15: No, from observations in Activity 1-2 it appears that this rule does not apply to bulbs. Bulbs behave differently than resistors.

Investigation 2:  Current Flow in parallel circuits

Equipment and Supplies

•	computer-based laboratory system

• 	two current sensors

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	a very fresh, alkaline 1.5-V D cell battery with holder

•	8 wires with alligator clip leads

•	3  #14 bulbs in sockets

•	contact switch

Overview

In this investigation students are introduced to parallel circuit connections.   Students discover properties of current flow in parallel circuits qualitatively (through observations of bulb brightness) and quantitatively through measurements with current sensors.

Question 2-1:  When the switch is closed, at each junction one terminal of one bulb is directly connected to one terminal of the other bulb, thus the bulbs are in parallel.  When the switch is open, there is no such connection at the top junction.

Activity 2-1: Brightness of Bulbs in a Parallel Circuit

In this activity, students investigate the current flow through a parallel circuit by observing bulb brightness in different configurations.

Question	 2-2:  When the switch is open, bulb D is bright and bulb E is not lit.  When the switch is closed, bulbs D and E are the same brightness, and their brightness is the SAME as bulb D when the switch was open.

Activity 2-2:  Current in Parallel Branches

In this activity, students investigate the current flow through a parallel circuit with current sensors.

| Figure L5-3: Graph of Current vs. Time for current entering 2 different bulbs in the parallel battery and 2-bulb circuit graphed for Activity 2-2 and Figure 5-7. |  |
| --- | --- |

Question 2-3:  Current through bulb D does not significantly change when the switch is closed.  (There may be a small decrease in current through bulb D when the switch is closed.  This decrease is due to the internal resistance of the battery in the circuit.) Current through bulb E is zero when the switch is open and is equal to the current through bulb D when the switch is closed.

Question 2-4:  Closing the switch does not significantly affect the current through bulb D. Evidence comes from brightness of the bulb as well as the measurements with the current sensor.

Activity 2-3:  Current Flowing Through the Battery

Students investigate the current flow from a battery when one and two bulbs are connected in parallel.

Question 2-5: In Figure 5-7 the current sensors measure current through each bulb. In Figure 5-8 CS1 measures current flow through bulb D but CS2 measures current flow from the battery. Sensor 2 measures the current through the battery because all of the current flowing through the battery must flow through it.

| Figure L5-4: Graph of Current vs. Time for current entering a bulb and current leaving the battery in the parallel battery and 2-bulb circuit graphed for Activity 2-3 and Figure 5-8. |  |
| --- | --- |

Question 2-6:  The current through a battery increases as the number of parallel bulb connections increases. If one considers that the bulbs provide some resistance to current flow, when one bulb is connected the current has only one path (with some resistance) through which to flow. When two bulbs are connected, the current now has two paths (with some resistance each) through which to flow.  etc., etc.

Question 2-7: The addition of another bulb in parallel decreases the total resistance.

Question 2-8: Even though a bulb with some resistance has been added, the significant effect is that another path for current flow has been added.

Question 2-9: We saw in Investigation 1 that when a bulb is added in series the current through the battery decreases. In this investigation we saw that when a bulb is added in parallel the current through the battery increases. Thus the amount of current through a battery appears to depend on both the arrangement of the bulbs and the number of bulbs in a circuit.

Question 2-10:  The current through a battery indicates the total resistance of the circuit connected to the battery. Thus both the number of bulbs and the arrangement of the bulbs are important in determining the total resistance of a circuit.

Investigation 3:  More complex series and parallel circuits

Overview

In this investigation students combine the knowledge they gained in Investigations 1 and 2 to analyze and investigate more complex circuits that contain both series and parallel connections. They also develop further their understanding of parallel and series connections through circuit diagramming exercises.

Question 3-1:  A is in series with the combination of B and C.

Question 3-2:  B and C are in parallel with each other.

Question 3-3:  The resistance of the combination of B and C is smaller than the resistance of B alone (current has two paths to flow through in the combination of B and C, therefore there is less resistance to current flow in that combination.)

Question 3-4:  The resistance of the combination of A, B, and C in 5-9(b) is smaller than the combination of A and B in 5-9(a) because the combination of B and C in 5-9(b) has smaller resistance than B alone. (See Question 3-3.)

Activity 3-1: A More Complex Circuit

Question 3-5:  When switch, S, is open, A and B have the same brightness. When the switch is closed, A is brightest. B and C are the same brightness, but not as bright as A. A becomes brighter when the switch is closed. When the switch is closed, bulbs B and C may not light as the potential difference across them (and current through them) may be too small. The relative brightness is still apparent with this result. (An un-lit bulb is dimmer than a glowing bulb.)

Data collected for this activity are shown in the table and the figure below:

|  | Switch Open | Switch Closed |
| --- | --- | --- |
| CS1 | 0.16A | 0.20A |
| CS2 | 0.17A | 0.12A |

| Figure L5-5: Graph of Current vs. Time for current through 2 different bulbs in the 3-bulb parallel and series circuit graphed for Activity 3-1 and the circuit in Figure 5-10 (b). |  |
| --- | --- |

Question 3-6:  The current through the battery and through bulb A increases when bulb C is added in parallel to B.  The resistance of the circuit has been reduced.

Extension 3-2:  An Even More Complex Circuit

Question E3-7:  A is parallel to the combination of B and C.

Question E3-8:  B is connected in series with C

Question E3-9:  The combination of B, C and D is parallel with A

Question E3-10: The combination of C and D is in series with B.

Question E3-11:  The current through branch 1 will not be affected by closing switch S or by unscrewing bulb B. The current through branch 2 will not be affected by unscrewing bulb A or by adding another bulb in series with bulb A.

Question E3-12:  In a circuit with two parallel branches connected across the battery, changes in one branch will not have an effect on the current through the other branch.

Extension 3-3:  Series and Parallel Networks

Question E3-13: 1 in series with 2;  3 in parallel with 4;  5 and 6 in series,  7 in parallel with combination of 5 and 6;  8, 9, and 10 in series.

Question E3-14:  Ranking from most resistance to least resistance: circuit number 5,2,1,4,3

Question E3-15:  Ranking from most current to least current:  circuit number 3,4,1,2,5.

Extension 3-4:  The Puzzler Problem

Question E3-16:  Bulb E brightest, bulbs A and B the same and second brightest, bulbs C and D the same and least bright.

Homework for Lab 5:  Current in Simple direct current circuits

1.	Which of the three circuits shown below, if any, are the same electrically?  Which are different?  Explain your answers.

All three circuits are the same electrically: two pairs of parallel bulbs connected in series with each other.

2.	Consider the two messy circuit diagrams #1 and #2 below.

A.	Identify which of the nice neat circuit diagrams (A, B, C or D) corresponds to circuit #1.  Explain the reasons for your answer.

Circuit B corresponds to circuit #1.

B.	Which circuit diagram (A, B, C or D) corresponds to circuit #2?  Explain the reasons for your answer.

Circuit D corresponds to circuit #2.

3.	Three of the circuits drawn below are electrically equivalent, and one is not.

A.	Which circuit is not like the others?  Explain how it is different.

Circuit C is different from the others.  It has 2 bulbs connected in series across the switch and battery, while circuits A, B, and D have two bulbs connected in parallel across the switch and battery.

B.	Which circuits represent parallel arrangements for the bulbs?  Which represent series arrangements?

See ‘A’ above

C.	In the boxes below, draw neat circuit diagrams for each of the arrangements.

4.	Use the model for electric current to rank the networks shown below in order by resistance. Explain your reasoning.

Ranked from lowest to highest resistance:  D, B, A,  E,  C

5.	If a battery were connected to each of the circuits in (4), in which case would the current through the battery be the largest? The smallest? Explain your reasoning.

Current through battery would be largest in circuit with smallest resistance: D. Current through battery would be smallest in circuit with largest resistance: C.

6.	The diagram below shows a typical household circuit. The appliances (lights, television, toaster, etc.) are represented by boxes labeled 1,2,3 and so on. The fuse, or circuit breaker, shown in the diagram is a switch intended to shut off the circuit automatically if the wires become too hot because the current is flowing in the circuit is too large.

Note: Although houses in the U.S. use alternating current (AC) which differs in some important ways from the direct current (DC) we have been studying, you can use the model you developed for these questions.

a.	What happens to the current through the fuse when more appliances are added to the circuit?  Describe evidence from this lab for your answer.

Current increases, as observed in Activity 2-3, parallel circuits.

b.	Does the current flowing through element 1 change when elements 2 and 3 are added to the circuit. Describe evidence from this lab for your answer.

The current through element 1 does not change (Activity 2-3).

c.	Is this model consistent with your observations of everyday household electricity?  For example, what happens to the brightness of a light bulb in a room when a second one is turned on?

The brightness stays the same (although the lights may dim momentarily when a motor turns on, but this is a more complicated effect).

d.	What may happen to the fuse if too many appliances are added to the circuit?  Why?

The fuse will ‘blow’.  That is, the current through the fuse will get so large that the fuse will break open and the power from the battery will no longer get to the elements in the circuit.

e.	What kind of circuit connection for elements 1, 2 and 3 is shown in the diagram?

Elements 1, 2, and 3 are connected in parallel with each other.

7.	Consider the circuit shown on the right

A.	Are the bulbs C, D and E connected in series, parallel or neither?  Explain.

The bulbs C, D, and E are in parallel.

B.	Rank the bulbs in order of brightness.  Use the symbols "=", "<" and ">".  Explain your ranking.

A=B >C=D=E   Current through A is the same as current through B .  Current through C=current through D= Current through E.  Total current through combination A and B must be same as total current through combination through C, D, E, therefore current through A and B individually is greater than current through C, D or E individually.

C.	How will the brightness of bulbs A and B change if bulb C is unscrewed?  Will the result be different if bulb D or E is unscrewed instead?  Explain.

A and B will become less bright.  The total resistance of the circuit increases when you unscrew C, therefore  the total current through the circuit decreases.  Thus the total current through A and B individually must decrease. The result will be the same regardless f whether C, D, or E is unscrewed.

8.	Consider the circuit shown on the right.  Rank the brightness of the bulbs in the circuit.  Use the symbols "=", "<" and ">".  Explain your ranking.

A>B>C>D=E

- In the two circuits below, the batteries and all bulbs are identical.  Compare the current flowing in the circuit on the left to the current flowing in the circuit on the right.  Be as quantitative as possible.

The current in the circuit on the right will be smaller than that in the circuit on the left. If the circuit elements are bulbs, then the current in the circuit on the right will be greater than 1/3 of the current in the circuit on the left.  It is not possible to be more quantitative than that.

- In the two circuits below, the batteries and all resistors are identical. Compare the current flowing in the circuit on the left to the current flowing in the circuit on the right.  Be as quantitative as possible.

The current in the circuit on the right will be 1/3 the current in the circuit on the left.

Is your answer the same as in question (9)?  Explain any differences.

Since resistors have a constant resistance, independent of the current flowing through them, it is possible to give a quantitative answer.  With the light bulbs, this is not possible.