SECTION L7: LAB 7—KirchHoff’s circuit Rules

General Introduction

In the following Investigations students develop an understanding of how to measure current, voltage and resistance with a digital multi-meter (DMM). In Investigation 1, they develop an understanding of the internal resistance values in a voltmeter and multi-meter. Students are introduced to the resistor color-code system and the tolerances of resistor values. They then explore two methods to measure resistance: the first is with the ohmmeter setting on the DMM, and the second is through deduction of resistance values from current and voltage measurements. In Investigation 2 students discover the rule for equivalent resistances in parallel and in series combinations of resistors and explore the use of the rule for several complex resistor combinations. In Investigation 3, students are introduced to circuit analysis with Kirchhoff’s Loop Rule and Kirchhoff’s Junction Rule, and they apply these rules to real circuits.

Lab 7 Equipment and Supplies List

• 	2 digital multi-meters (one with resistance mode)

• 	2 very fresh, alkaline 1.5-V D cell batteries with holders

• 	6 alligator clip leads

• 	2 #14 bulbs and sockets

•	several color-coded resistors

•	3  51Ω carbon resistors

•	3  22 Ω carbon resistors

• 	100 Ω resistor

•	2 resistors (rated values of 39 Ω and 75 Ω)

•	6-V battery (very fresh, alkaline)

•	0-200 

Investigation 1: Batteries and Voltages in Series Circuits

Equipment and Supplies

• 	2 digital multi-meters

• 	2 very fresh, alkaline 1.5-V D cell batteries with holders

• 	6 alligator clip leads

• 	2 #14 bulbs and sockets

•	several color-coded resistors

Overview

Students learn how to connect a multi-meter to measure current, voltage and resistance.  In particular the students learn how to connect the sensors to measure current or voltage and learn how the internal resistance of the sensor in combination with the connection serves to minimize the interference of the meter with the currents and voltages in the circuits that they are measuring.

Activity 1-1:  Measuring Current with a Multi-meter

Students should initially connect the circuit without the DMM connected so that they can see the brightness of the bulbs.  The bulbs should be lit and equally bright in this initial condition.

Question 1-1:  Figure 7-2(b) is correct. When the meter is connected like Figure 7-2(b), the currents through both bulbs and the meter are the same. The brightness of both bulbs is the same as in the initial condition (Figure 7-2(a)). When the ammeter is connected as in Figure 7-2(c), very little current flows through bulb 1. Bulb 1 does not shine at all and bulb 2 gets brighter.

Question 1-2:  The multi-meter set as an ammeter behaves as if it were a small resistor. The voltage drop across the ammeter is small so that the net voltage across bulbs 1 and 2 is almost unchanged from when the ammeter was not connected.

Activity 1-2:  Measuring Voltage with a Multi-meter

With the multi-meter set as a voltmeter connected incorrectly (as in 7-3(a)), the bulbs do not light and the voltmeter reads 3.0 volts, like the voltage across the battery.

Question 1-3:  The correct connection is as in 7-3(b). The voltage across the voltmeter is the same as the voltage across bulb 1 in that case because the voltmeter is in parallel with the bulb.

Question 1-4: The voltmeter acts as a large resistor. A very large resistance results in a very small current through the voltmeter, thus, the current through bulbs 1 and 2 and through the battery will be essentially unchanged.

Activity 1-3:  Reading Resistor Codes and Measuring Resistance

Students use resistor color-codes to read values of carbon resistors. They measure resistances of the resistors in two ways: first, by measuring with the ohmmeter setting on the DMM, and second, by measuring current through and voltage across the resistor when it is connected to a battery.  Students thus investigate measurement of resistors and precision of resistors.

Note that at 10 ohms, the current gets relatively large and the resistor heats up significantly when connected to a 6-V battery.  The resistance therefore is less stable than for higher values of resistors.  We recommend using resistor values of at least 100.

Table 7-2

| R from code ( | Tolerance from code | Measured R ( | Measured V (V) | Measured I (A) | R = V/I ( |
| --- | --- | --- | --- | --- | --- |
| 560  | 5 % | 573  | 5.6 V | 9.6 mA | 583  |
| 1000  | 5 % | 988  | 5.6 V | 5.6 mA | 1000  |
| 10  | 5 % | 10  | 4.5 V | 440 mA | 10  |
| 220,000  | 5 % | 218,000  | 5.6 V | 2.58 A | 217,000  |

Question 1-5:  All measured values are within 5% of the coded value.

Question 1-6:  Measured values with the ohmmeter are within 2% of measured values through measuring voltage and current.

Investigation 2:  Series and parallel combinations of resistors

Equipment and Supplies

•	3  51Ω carbon resistors

•	3  22 Ω carbon resistors

•	75 Ω  resistor

• 	digital multi-meter

• 	6 alligator clip leads

Overview

Students observe equivalent resistances of resistors combined in series and parallel and generate a general model for equivalent resistances of resistor combinations for each case.

Activity 2-1:  Equivalent Resistance of Resistors Connected in Series

Actual resistor values (measured in resistance mode on DMM):

R1 53.1Ω	R2 53.1Ω	R3 53.7Ω

Resistance of R1 and R2 in series:     	106 

Resistance of R1, R2 and R3 in series:  160 

Question 2-1:  Answer depends on student prediction.

Question 2-2: Req=R1+R2+R3 = 53.1 + 53.1 + 53.7 = 159.9Ω

Question 2-3:  The rule is consistent with that developed in Lab 5, namely when two identical resistors are connected in series, the resistance is twice as large, and the current through them is half as large.

Activity 2-2:  Equivalent Resistance of Resistors Connected in Parallel

Actual resistor values (measured in resistance mode on DMM): Resistance of R1 and R2 in parallel: 	26.7 

Resistance of R1, R2 and R3 in parallel: 17.9 

Question 2-4:  Answer depends on student prediction.

Question 2-5:

1/ Req  =  1/ R1  +  1/ R2  +  1/ R3  = 1/53.1 + 1/53.1 + 1/53.7 = 0.05629

Req  =  17.8The results are fully consistent with the proposed model.

Question 2-6:  Yes, the relationship is consistent with Lab 5, namely, the current through the battery was twice as large when two identical resistors were connected in parallel.

Extension 2-3: Other Combinations of Resistors in Series and Parallel

R4 22.3Ω	R5 75.2Ω

Question E2-7:  Series combination of three different resistors :

Measured value is 151 		Req  = 53.1 +22.3 +75.2 = 150.6

Actual resistor values (measured in resistance mode on DMM): Resistance of R1, R4 and R5 in parallel: 13.2

Question E2-8:

1/ Req  =  1/ R1  +  1/ R4  +  1/ R5  = 1/53.1 + 1/22.3 + 1/75.2 = 0.07697       Req  =  13.0, completely consistent with the measured value.

Extension 2-4:  The Equivalent Resistance for a Network

Question E2-9:  Using R1 = 22 and R2 = 51the calculated value is 15.2The measured value was 16 .

Investigation 3: Kirchhoff's Circuit RULES

Equipment and Supplies

• 	2 resistors (rated values of 39 Ω and 75 Ω)

• 	digital multi-meter (to measure resistance)

•	6-V battery

•	very fresh, alkaline 1.5-V D cell battery

• 	100 Ω resistor

•	0 to 200 

•	6 alligator clip leads

Overview

Students apply Kirchhoff’s laws to a circuit. They then check their theoretical calculations by measuring (with a digital multi-meter) the voltages and currents in the actual circuit. In the Extension students compare the analysis of the relatively complex circuit they studied in Activities 3-1 and 3-2 with a simpler (1-loop) circuit.

Activity 3-1: Applying the Loop and Junction Rules Several Times

Measured values of the two fixed resistors of 75Ω and 39Ω , and the two battery voltages (measured with multi-meter).

Measured value of the 6-V battery emf 1:5.73 V

Measured value of the 1.5-V battery emf 2:1.54V

Measured value of the 75Ω resistor R1:75.2

Measured value of the 39Ω resistor  R3:49

Question 3-1: They are not in series because there is more than one path for the current I2 at Junction 2, so the same current doesn’t necessarily flow through R1 and R2. They are not in parallel because only one set of terminals is connected together. The other connection is through battery 1.

Question 3-2: Substitutions should result in identical equations.

Question 3-3: Answer depends on student measurements.

Activity 3-2: Testing Kirchhoff's Laws with a Real Circuit

|  | Measured R (ohms) | Measured V (volts) | Measured I=V/R (milliamps) | Calculated I (milliamps) | % Difference |
| --- | --- | --- | --- | --- | --- |
| R1 | 75.2  | 3.1 V | 41 mA | 44 mA | 7% |
| R2 | 100  | 2.4 V | 24 mA | 25 mA | 4% |
| R3 | 49  | 0.85 V | 17.3 mA | 19 mA | 10% |

Question 3-4: Answer depends on student measurements. For the above measurements, 44 mA = 25 mA + 19 mA

Question 3-5: Answer depends on student measurements. Going around the loop in a counterclockwise direction as indicated, starting at the battery,                        -5.73 V + 3.1 V +2.4 V = -0.2 V,

or zero within the accuracies of the measurements.

Question 3-6: They are all within 10%.

Extension 3-3:  How Do Changes Affect the Currents?

| Picture of the modified circuit (with R2 removed). |  |
| --- | --- |

Question E3-7:  The analysis of the modified circuit will be simpler: only one loop rule must be executed and no junction rules.  There is only one loop and one current in the circuit!

Question E3-8:  Effective value of R2 when R2 is removed is infinite.  There is no possibility of current flow, which is equivalent to an infinite resistance in that branch.

Measurement of  I1 and I3 with R2 removed:

V1:    2.39V   		V3:  1.54V

I1:   2.39V/75.2=31.8mA

I3:   1.54V/49W =  31.4mA

Question E3-9:  Answer depends on student prediction.

Homework for Lab 7: KIRCHHOFF'S CIRCUIT LAWS

1.	Find the equivalent resistance of the following network.  (All resistances are in ohms.)  Show your work below.

a) Two 10 resistors in series: RA= R1+R2=10+10=20

b) RA in parallel with R3:  1/RB=1/RA+1/20, so  RB = 10

c) R4  in series with RB:  20 + 10 = 30.

d) R6  in parallel with R8:  1/RC = 1/20 + 1/30, so RC= 12

e) R5 in series with Rc and R7:  RD = 10 + 12 + 8 = 30=RD

f) RD in parallel with RB:  1/Req = 1/RD + 1/RB = 1/30 + 1/30, so Req = 15

2.	Show on the circuit diagram in Question 1 how you would connect a multi-meter to measure the current through the 8- resistor.

a.	Explain why the multi-meter is connected in this way.

The current that flows through the 8-ohm resistor also flows through the multi-meter when it is connected as shown.

b.	What design feature of a good multi-meter allows you to connect it in the way indicated without appreciably affecting the current through the 8- resistor?

The multi-meter has a very small internal resistance. Therefore even with current flowing through it, only a small voltage drop exists across it.  Thus, in the circuit it acts almost like an ideal wire.

3.	Show on the circuit diagram in Question 1 how you would connect a multi-meter to measure the voltage across the 8- resistor.

a.	Explain why the multi-meter is connected in this way.

The multi-meter measures the voltage across the 8 resistor.  In the arrangement shown, the multi-meter is in parallel with the 8 resistor, thus the voltage across the multi-meter is the same as the voltage across the 8resistor.

b.	What design feature of a good multi-meter allows you to connect it in the way indicated without appreciably affecting the voltage across the 8- resistor?

The multi-meter has a very large internal resistance. Therefore, even when connected in a circuit, only a small current flows through it.  Thus it appears in a circuit as if it were an ‘open circuit’—as if it were not in the circuit.

| 4. A battery with emf 12 V and internal resistance 1 is pictured in the thick-lined box in the circuit on the right. A is the positive terminal of the battery and B is the negative terminal. |  |
| --- | --- |

a.  What is the potential difference across the battery’s terminals when the switch S is open, as shown?

12 V

b.  What is the potential difference across the battery’s terminals when the switch S is closed?

I(loop) = 12V/16 = 750mA.   Thus VA-B = 12V - (750mA)1 = 11.25V.

5.	Give the values and tolerances of resistors with the following color codes:

a.	red-red-brown-gold  220  5%

b.	violet-grey-blue  78M,  20%

c.	Could the value of a resistor marked as (a) actually be 240 ?  Explain.

No, the 220 resistor has a 5% tolerance, so its value should be in the range 209-231.

| 6. Find the current through each of the resistors in the circuit on the right. Give both the magnitude and direction of current flow. Show all work below. |  |
| --- | --- |

Kirchhoff’s Loop Rule: Loop A:  3V-I13 - 12V - I26 = 0

Kirchhoff’s Loop Rule: Loop B:  3V-6V-I32 - I26 = 0

Kirchhoff’s Junction Rule:  I1 + I3 =I2

Solving the three equations simultaneously: I1 = -1.5A; I3 =0.75A; and I2 =-0.75A