SECTION L6: LAB 6—Voltage in Simple DC Circuits and Ohm’s Law

General Introduction

These investigations lead students to develop a more complete model for potential difference in circuits.  The students investigate the relationship between the voltage supplied by the battery and the potential drops across elements in series and in parallel circuits.  The students then investigate the relationship between resistance, current and potential difference.   These investigations lead students to discover Ohm’s law, which relates current through a resistor to the voltage across it.

Lab 6 Equipment and Supplies List

•	computer-based laboratory system

•	two voltage sensors

•	current sensor

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	2 very fresh, alkaline 1.5-V D cell batteries and holders

•	6 wires with alligator clip leads

•	4  #14  bulbs in sockets

•	2 contact switches

• 	variable regulated DC power supply (up to 3 V and 0.5 amps)

• 	10-ohm resistor

Investigation 1: Batteries and Voltages in Series Circuits

Equipment and Supplies

•	computer-based laboratory system

•	two voltage sensors

•	current sensor

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	2 very fresh, alkaline 1.5-V D cell batteries and holders

•	6 wires with alligator clip leads

•	4  #14  bulbs in sockets

•	contact switch

Overview

Students explore voltages across batteries and across bulbs in simple series circuits.  Students develop the model that the voltages across elements connected in series add. Students also investigate the voltages across batteries connected in series and parallel.

Activity 1-1:  Battery Action

The batteries should be fresh for this experiment so that the voltages are close to the specified values and so that voltages of A and B are similar.

Question 1-1:  The bulbs should have the same brightness.

Question 1-2: The current in the two-bulb, two-battery circuit is the same as the current in the one-bulb, one-battery circuit.

Question 1-3: The resistance of a circuit increases as more bulbs are added in series. (This was discovered in Lab 5.) You must increase the voltage applied to the circuit.

Question 1-4: The bulb is brighter in the one-bulb, two-battery circuit than in the one-bulb, one-battery circuit.

Question 1-5:  Increasing the number of batteries connected in series, increases the current in a series circuit.

Activity 1-2: Batteries in Series and Parallel

Sensors should be zeroed before students begin measurements.

Voltage of battery A:   1.51V	  Voltage of battery B:  1.52V

Question 1-6: The voltages may be somewhat higher or lower than the marked values, depending on how fresh the batteries are.

Voltage of battery A:   1.51V	  Voltage of A and B in series:  3.02V

Question 1-7: The voltage across battery A is the same as that from Question 1-6; the voltage across A and B in series is the sum of the individual voltages of A and B.

Voltage of battery A:   1.51V      Voltage of A and B in parallel:  1.52V

Question 1-8: The voltage across A and B in parallel is the same as the individual voltages of the batteries.

Question 1-9: The combined voltage for batteries connected in series is the sum of the voltages of the individual batteries.

Question 1-10:  The voltage across a set of identical batteries connected in parallel is the same as the voltage across one of the batteries alone.

Activity 1-3: Voltages in Series Circuits

Sensors should be zeroed before measurements begin.

The data taken from the measurements are shown below:

Potential difference across the battery:  1.46V

Potential difference across bulbs A and B in series:  1.42V

Question 1-11: The voltage across the battery is identical to the voltage across the two bulbs. Any difference is due to calibration or zeroing differences.

The data taken from the measurements are shown below:

Potential difference across bulb A:  0.72V

Potential difference across bulb B:  0.71V

Question 1-12: For identical bulbs, the voltages across each bulb individually should be the same. Also, the sum of the voltage across A and the voltage across B is exactly the measured voltage across the battery measured earlier.  Again, differences are due to calibration or zeroing differences.

Question 1-13: The sum of the voltage across bulb A and the voltage across bulb B gives you the voltage across the two bulbs combined in series. For this series circuit, the sum of the individual voltages of the two bulbs also gives you the voltage across the battery.

Investigation 2:  Voltages in Parallel Circuits

Equipment and Supplies

•	computer-based laboratory system

•	two voltage sensors

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	very fresh, alkaline 1.5-V D cell batteries (must be very fresh, alkaline) and holders

•	1.5-V D cell battery that is not fresh

•	6 wires with alligator clip leads

•	3  #14  bulbs in sockets

•	2 contact switches

Overview

Students explore the potential differences across elements in a parallel circuit and develop a general rule from their observations.

Activity 2-1:  Voltages in a Parallel Circuit

It is important that students zero their sensors before taking measurements.

The data extracted from the graph are shown below:

Switch open:

Voltage across battery: 1.45V	Voltage across bulb A: 1.42V

Switch closed:

Voltage across battery: 1.39V	Voltage across bulb A:  1.35V

Question 2-1: Student attention should be focused on voltage measurements in the steady state (neglecting any transient spikes that happen just as the switch closes).  The voltage across the battery and across bulb A is not significantly affected when the switch is switched from open to closed.   (Some small change in voltage is due to the internal resistance of the battery. This will be studied in Activity 2-2. The internal resistance can be minimized by using fresh batteries.)

| Figure L6-1: Voltage vs. Time in a battery and 2-bulb circuit. Voltage 1 is across the battery and Voltage 2 is across bulb A. Graphed for Activity 2-1. |  |
| --- | --- |

| The data extracted from the graph are shown below: Figure L6-2: Voltage vs. Time in a battery and 2-bulb circuit. Voltage 1 is across bulb A and Voltage 2 is across bulb B. Graphed for Activity 2-1. |  |
| --- | --- |

Switch open:

Voltage across bulb A: 1.51V	Voltage across bulb B: 0V

Switch closed:

Voltage across bulb A: 1.43V	Voltage across bulb B:  1.41V

Question 2-2:  The voltage across bulb A is not significantly affected when the switch is closed. This is consistent with Question 2-1.

Question 2-3: The voltage across bulb B increased from zero to about 1.41 volts (the battery voltage) when the switch was closed. A potential difference exists across the bulb when the bulb terminals are connected to opposite terminals of the battery.

Question 2-4: The potential differences across individual elements connected in parallel are equal. When the terminals of the parallel connection are connected to the terminals of a battery, the potential difference across the parallel combination and across the individual elements is the same as the potential difference across the terminals of the battery.

Question 2-5: The battery is a constant voltage source.  In Activity 1-3 and Activity 2-1 we saw that the amount of current from a battery changes significantly when the circuit connected to it changes, but the voltage across a battery does not change much when the circuit connected to it changes.

Question 2-6: The potential difference (voltage) is zero.

Activity 2-2:  Internal Resistance of a Battery

Question 2-7: The voltage across the battery decreased as the current drawn from the battery increased.

Question 2-8: Yes, the potential difference across the internal resistance, R, subtracts from the potential difference across the battery.

Extension 2-3: Applying Your Current and Voltage Models

Question E2-9: The current that flows through bulb A splits into current through bulb B and C, so bulb A will be brightest. Bulbs B and C are in parallel therefore the voltage across them is identical and the current through them is identical. The brightness of bulbs B and C is therefore identical. Bulbs B and C are less bright than bulb A.

Investigation 3:  Ohm’s Law

Equipment and Supplies

•	computer-based laboratory system

•	current and voltage sensors

•	RealTime Physics Electricity and Magnetism experiment configuration files

• 	variable regulated DC power supply (up to 3 V and 0.5 amps)

•	6 alligator clip leads

• 	#14 bulb in a socket

• 	10-ohm resistor

Overview

In this investigation students study the relationship between potential difference and current for a bulb and for a resistor.

Activity 3-1: Current and Potential Difference for a Resistor

Question 3-1: The current increased as the power supply voltage was increased.

Question 3-2: The potential difference across the resistor increased as the current through the resistor increased.

Question 3-3: The current is proportional to the potential difference, V=IR.

| Figure L6-3: Current vs. Time and Voltage vs. Current for a circuit with a DC Power Supply and a resistor. Voltage 2 is across the resistor and Current 1 is through the resistor. Graphed for Activity 3-1. |  |
| --- | --- |

Question 3-4: The resistance of a resistor appears to be constant.

Question 3-5: The value should be within 10 percent of the 10 specified value.

Extension 3-2 Relationship Between Current and Potential Difference for a Light Bulb

| Figure L6-4: Current vs. Time and Voltage vs. Current for a circuit with a DC Power Supply and a bulb. Voltage 2 is across the bulb and Current 1 is through the bulb. Graphed for Extension 3-2. |  |
| --- | --- |

Question E3-6: The bulb got brighter as the power supply voltage got larger.

Question E3-7: The larger the potential difference, the brighter the bulb. Larger potential difference also means larger current.

Question E3-8:  In both cases (bulb and resistor) the voltage increases as the current increases.  In the case of the resistor, the voltage vs. current plot is linear and goes through (0,0), indicating that the voltage and current are proportional to one another in the resistor. In the case of the bulb, the plot is not linear.

Question E3-9: So the relationship is not proportional.

Question E3-10: The value of R for a bulb changes as the current through it changes. (This is not the case for a resistor.)

Question E3-11: The light bulb is not an ohmic device since its resistance is not constant.

Homework for Lab 6:  Voltage in Simple DC Circuits and Ohm’s Law

1.	In the circuit below, the battery maintains a constant potential difference between its terminals at points 1 and 2 (i.e., the internal resistance of the battery is considered negligible).

The three light bulbs, A, B and C are identical.

a.	How do the brightnesses of the three bulbs compare to each other?  Explain your reasoning.

A > B = C.  The potential difference across A and BC is the same but since BC has a greater resistance, the current in BC will be less than in A.  Hence the brightness of A will be greater.

b.	What happens to the brightness of each of the three bulbs when bulb A is unscrewed and removed from its socket?    Explain your reasoning.

A goes out - no connection.

B and C remain the same.  The two branches are independent so changes in branch A do not effect branch BC.

c.	What simultaneously happens to the current through points 3, 4 and 5?  Explain your reasoning.

The current through point 3 decreases.  Since there is no longer any current in branch A there is less current through the battery.

The current through point 4 remains unchanged since the individual branches A and BC are independent.

The current through point 5 becomes zero since there is no longer a complete circuit in branch A.

d.	Bulb A is screwed back in.  What happens to the brightness of each of the three bulbs when bulb C is unscrewed and removed from its socket?  Explain your reasoning.

Bulb A remains the same since the branches are independent.

Bulbs B and C go out since there is no longer a complete circuit for current to flow in branch BC.

e.	What simultaneously happens to the current through points 3, 4 and 5?  Explain your reasoning.

The current through point 3 decreases because current no longer flows in branch BC and the current through point 3 is the sum of that in branches A and BC.

The current through point 4 is zero since there is no longer a complete circuit in branch BC for current to flow.

The current through point 5 remains unchanged since the branches A and BC are independent.

2.	For each of the questions a-e below, a wire is connected from the battery terminal at point 1 to point 4.

a.	What happens to the brightness of each of the three bulbs? Explain.

The brightness of bulb A remains unchanged since the branches A and BC are independent.

Bulb B goes out since the wire from point 1 to point 4 forces both sides of bulb B to be at the same potential--no potential difference means no current flow.

The brightness of bulb C will increase since the potential difference across it alone is now equal to the potential of the battery - so more current will flow.  C = A

b.	What happens to the current through point 3? Explain.

The current through point 3 decreases to become equal to that through point 5.  None of the current through bulb C flows through point 3 now.

c.	What happens to the potential difference across bulb B? Explain.

The potential difference across bulb B is zero.  Placing the wire from point 1 to point 4 forces the potential at both sides of B to be the same so there is no potential difference and no current through B.

d.	What happens to the potential difference across bulb C? Explain.

The potential difference across bulb C increases to be the same as that of bulb A and that of the battery--it doubles.

e.	What happens to the potential difference between points 1 and 5? Explain your reasoning.

The potential difference between points 1 and 5 remains unchanged since branch A is connected directly to the battery and is independent of branch BC.  We have only changed branch BC.

3.	The wire described in (2) is removed. What happens to the brightness of each of the three bulbs and to the current through point 2 if a wire is connected from the battery terminal at point 2 to the socket terminal at point 5?

All bulbs and currents remain unchanged.  This simply is adding another wire where one already exists and points 2 and 5 and the right-hand side of C are already at the same potential.

- The circuit is returned to its original state.  A fourth bulb (D) is connected in parallel with bulb B (not in parallel with B & C).

a. Sketch the bulb in the circuit.

b.	What happens to the brightness of each of the three bulbs?

Bulb A remains unchanged since branch A is independent.

Bulb B gets dimmer since the current is now split in two with half the current in the branch flowing through bulb B and half the current flowing through bulb D.

Bulb C gets brighter since adding a bulb in parallel reduced the total resistance in the branch so the total current through the branch increases and all of the current must flow through bulb C.

c.	What happens to the current through point 3?

The current through point 3 increases since the total resistance of the circuit is decreased by decreasing that in branch BCD.

d.	What happens to the potential difference between points 3 and 4?

The potential difference between points 3 and 4 is decreased.  The total potential drop across BCD is equal to that of the battery but now the current through bulb C is increased.  The potential drop across bulb C  =  IR is increased and so the potential drop across points 3-4 (bulbs B & D) must be decreased.

e.	What happens to the potential difference between points 4 and 2?

The potential difference across points 4 and 2 increases.  Since the current through bulb C increases the potential drop across bulb C = IR must also increase.

5.	State Ohm's law in words.  For what type of circuit elements does it correctly describe the behavior?

The current through a resistor is proportional to the potential difference across the resistor and inversely proportional to the resistance.  This does not apply to light bulbs since their resistance depends on the current through them.

- Does a light bulb have a constant resistance?  Explain.  Is a light bulb ohmic?

No, the resistance of a light bulb changes as the current changes.  The graph of voltage vs. current for the light bulb did not have a constant slope.  A light bulb is not ohmic.

- Does a resistor have a constant resistance?  Explain.  Is a resistor ohmic?

Yes, the resistance of a resistor is constant as the current changes.  The graph of voltage vs. current for the resistor had a constant slope.  A resistor is ohmic.

8.	Draw diagrams for a 75  and a 100  resistor connected in series and connected in parallel:

SERIES:                                                           PARALLEL:

9.	In the following circuits, tell which resistors are connected in series, which are connected in parallel and which are neither in series or parallel.