SECTION L4:  LAB 4—Batteries, Bulbs and Current

General Introduction

The investigations in Lab 4:  Batteries, Bulbs and Current guide students to begin to develop an understanding of the relationship between potential difference and current flow in conductors.

In Investigation 1, students are introduced to the concept of sources of potential difference and observe charge flow through the discharging of two parallel plates and a conducting ball between them. Students further investigate charge flow through exercises with batteries, bulbs and switches. The lighted bulb serves as a concrete indicator to students about whether charge is flowing, and the battery/bulb/switch exercises give students concrete experience with ‘open’ and ‘closed’ circuits. In Activity 1-5, students are introduced to microcomputer-based sensors for measuring current flow. With these exercises the relationships between the intensity of the lighted bulb, potential difference and current flow are developed. Finally, students develop a model for current flow and confront a common and firmly held misconception about current being ‘used-up’ as it travels through a circuit.

In Investigation 2, students develop the concept of potential difference and observe the relationship between current flow and potential difference in a simple circuit. In Activity 2-1 students are introduced to conventional symbols for switches, bulbs, batteries, etc., and draw diagrams for several circuits. In Activity 2-2 and Extension 2-3, students explore circuits further by designing and constructing ‘functional’ circuits such as Christmas tree lights, lights for a tunnel, or entry and exit switches. Students are then introduced to microcomputer-based sensors for measuring potential difference in Activity 2-4. Students observe potential difference (voltages) across the elements in a simple battery/switch/bulb circuit. An important observation that the students make is the constant potential difference across the battery regardless of whether the switch is open or closed and regardless of how many bulbs are in the circuit. Finally, students measure and observe the potential difference and current in the circuit as the switch is opened and closed.

In Investigation 3, students are introduced to a microscopic model for current flow in a circuit. They are then led to develop analogical models for batteries, current, electric charge and electric energy in a circuit based on a two-dimensional mechanical analog.

Lab 4 Equipment and Supplies List

• 	2 metal angle irons (approx. 15 cm long)

• 	foil-covered Styrofoam ball on a string (2.5 cm diameter.)

• 	300-volt battery pack or power supply

• 	hard rubber (plastic or Teflon) rod and fur

• 	glass or acrylic rod rod and polyester, felt or silk cloth

• 	electroscope

• 	alligator clip leads

• 	Wimshurst or Van de Graaff generator (optional)

• 	flashlight bulb (#14)

• 	flashlight battery (1.5 V. D cell, alkaline and very fresh)

•	wire (6 inches or more in length)

•	3 #14 bulbs and sockets

• 	common objects (paper clips, pencils, coins, rubber bands, fingers, paper, keys etc.)

• 	battery holder (for a D cell)

• 	several wires (6 inches or more in length)

• 	flashlight bulb socket

• 	contact switch

•	computer based laboratory system

•	two current sensors

•	two voltage sensors

•	6 alligator clip leads

•	single-pole-single-throw (SPST) switch

•	single-pole-double-throw (SPDT) switch

•	double-pole-double-throw (DPDT) switch

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	mechanical model for current flow in a circuit (optional)

Current sensors: Current sensors are available from Vernier (DCP-BTA) for the LabPro or LabQuest interface and from PASCO (CI-6556) for the Science Workshop interfaces. The Vernier sensors have an internal resistance of 0.1 ohm which is small compared to the cold resistance of a #14 flashlight bulb (about 10 ohms). However, the PASCO sensors have an internal resistance of 1 ohm. If using the PASCO sensors, to avoid interference from the sensor’s resistance, it is advisable to use 6 V lantern batteries and bulbs in place of the 1.5 V batteries and bulbs called for in these activities.

Voltage sensors: Voltage sensors are also available from both companies, the Vernier Differential Voltage Sensor (DVP-BTA) and PASCO Voltage Sensor (CI-6503).

Investigation 1:  Models describing current flow

Equipment and Supplies

• 	2 metal angle irons (approx. 15 cm long)

• 	foil-covered Styrofoam ball on a string (2.5 cm diameter.)

• 	300-volt battery pack or power supply

• 	hard rubber (plastic or Teflon) rod and fur

• 	glass or acrylic rod rod and polyester, felt or silk cloth

• 	electroscope

• 	alligator clip leads

• 	Wimshurst or Van de Graaff generator (optional)

• 	flashlight bulb (#14)

• 	flashlight battery (1.5 V. D cell, alkaline and very fresh)

•	wire (6 inches or more in length)

• 	common objects (paper clips, pencils, coins, rubber bands, fingers, paper, keys etc.)

• 	battery holder (for a D cell)

• 	several wires (6 inches or more in length)

• 	flashlight bulb socket

• 	contact switch

•	computer based laboratory system

•	two current sensors

•	RealTime Physics Electricity and Magnetism experiment configuration files

Overview

This investigation introduces students to current flow, which is the flow of electric charges, in conductors.  Students first observe the flow of electric charges in the discharging of an electroscope. They then are introduced to microcomputer-based sensors to measure current.  With the sensors students observe current flow in simple battery, bulb and switch circuits.

Activity  1-1:  Comparing Stuff from a Battery to the Rubbing Stuff

Humidity decreases the quality of the effect.  While doing this lab attempt to minimize room humidity as much as possible.

Question	1-1: The foil should have flipped away from the vertical metal bar since each is now charged with ‘like’ charges, which repel.

Question 1-2: The ball initially is attracted to one or the other plate.  When the ball touches the plate, it becomes charged with the polarity of the plate to which it was attracted.  Once it is charged, it is repelled by that plate and attracted to the other (oppositely charged plate).   When it touches that plate, the ball becomes charged like the one it is touching, at which time it is repelled by that plate, and so on, and so on.  The swinging back and forth eventually slows down and stops.  This occurs when the plates have been discharged.

Question 1-3:  A power supply will probably work better than a battery.   The electroscope had the same effect.  The ball bounces between the plates, similar to the charging with rubbing method.  If the battery is connected to the plates continually during the demonstration, the swinging of the ball does not stop since the power supply continually supplies charges to the plates.

Question 1-4: The Van de Graaff has the same effect on the electroscope.  If connected continually during the demonstration, the ball will continually swing between the charged plates, as with the power supply.

Question 1-5: The charges cause the same effect and appear to be the same thing.

Activity 1-2:  Arrangements that Cause a Bulb to Light

Research shows that many students do not know how to connect a bulb to make it light.  Students have difficulties with the location of the connections as well as with the fact that a circuit must consist of a closed loop of connections.

Question 1-6:  Sketches should clearly show a closed circuit and should show one end of the battery connected to the bottom of the bulb and the other end connected to the metallic material on the side of the bulb.

Question 1-7:  Open circuits or connections where there is not a complete circuit through the bulb from one end of the battery to the other.

Question 1-8:  A closed circuit is required: connections from one end of the battery to one ‘terminal’ of the bulb and from the other ‘terminal’ of the bulb to the other end of the battery makes a completed circuit (a closed loop).

Activity 1-3:  Other Materials Between the Battery and Bulb

The observations of circuits which include different objects show that a variety of materials conduct and a variety are non-conductors, or insulators.

Question 1-9:  Student answers will depend on the selection of materials available to them.

Question 1-10:  Again, student answers will vary.

Question 1-11:  Metallic materials seem to be conductors of electricity; non-metallic materials do not conduct electricity.

Activity 1-4:  Using a Battery Holder, Bulb Socket, and Switch.

Question 1-12:  The filament connections are useful for a number of reasons.     1) When both ends of the filament are connected to the socket and the circuit is completed, current flows through the conducting filament and the bulb lights.      2) The closed circuit is easily opened if needed by unscrewing the bulb just enough to open the contact between the bottom of the bulb and the socket.

Question 1-13: Students apply the concepts of closed and open circuits to the bulb socket.

Question 1-14:  Part of the electrical energy from the battery is changed into heat energy as it flows through the circuit.  (Some of the electrical energy is changed into light energy.)

Question 1-15:  Students again develop and confirm the requirement of closed circuit for current flow.

Activity 1-5:  Developing a Model for Current Flow in a Circuit.

Current sensors should be zeroed before measurements begin.

Figure L4-1: Current vs. Time for battery and bulb circuit for Activity 1-5.

Question 1-16:  Graphs should show that the currents are practically identical.  To check that the sensors contribute differences and not the circuit, switch the current sensors with each other and re-measure.  If the difference switches with the sensors, it is caused by them.

Question 1-17:  If a difference exists, check that sensors have been zeroed.  If differences persist, calibrate sensors.  This is an unlikely event.

Question 1-18: Model D appears to be the correct one: Neither the magnitude nor the direction of current flow changes after going through the bulb.

For the other, incorrect models:

Model A: Observations show that current exists in both wires, therefore this model does not apply

Model B: This model appears to be correct magnitude-wise, but the direction of current flow in one of the wires should be switched.

Model C: The current was equal in both wires, so this model does not apply.

Investigation 2:  Current and Potential Difference

Equipment and Supplies

•	3 #14 bulbs and sockets

•	1.5-V D-cell very fresh, alkaline battery with holder

•	6 alligator clip leads

•	single-pole-single-throw (SPST) switch

•	single-pole-double-throw (SPDT) switch

•	double-pole-double-throw (DPDT) switch

•	computer-based laboratory system

•	two voltage sensors

•	two current sensors

•	RealTime Physics Electricity and Magnetism experiment configuration files

Overview              							            This investigation advances student understanding of circuit concepts and current flow with exercises of building circuits and drawing circuit diagrams.  The students are then introduced to the concept of potential difference.  Students measure potential difference at several points in a simple circuit with microcomputer-based voltage sensors.  Students then measure current through a battery/bulb/switch circuit while the switch is opened and closed.  During this exercise students also observe the voltage across the battery as the switch is opened and closed.

Activity 2-1: Drawing Circuit Diagrams

Circuit Sketch A			Circuit Sketch B

Question 2-1: The long line represents the positive terminal of the battery. (Compare the two representations in  Figure 1-10.)

Activity 2-2: Inventing and Constructing Electric Circuits

Extension 2-3: Inventing Other Circuits

Question 2-2: Student answers will vary. Important points are that it should be clear from the diagrams which components are used in the circuit and where wires are connected to the components.

The actual diagrams drawn by the students may appear somewhat different than those shown.  The connections shown give the functionality asked for in the descriptions.

| Circuit for Christmas Tree Lights |  |
| --- | --- |
| Circuit for Lighting a Tunnel |  |
| Circuit for Exit and Entry Light Switches |  |

Activity 2-4: Measuring Potential Difference (Voltage) and Current

The voltage sensors should be zeroed before the measurements begin.

Figure L4-2: Voltage vs. Time for battery and bulb circuit for Activity 2-4.

Question 2-3: The voltage is zero when the leads of the voltage sensor are connected to each other.

Question 2-4:  The voltage across the battery does not change when the switch is opened and closed. The voltage across the bulb is zero when the switch is open and it is equal to the battery voltage when the switch is closed. The explanation for the battery: the battery always supplies the same voltage. The explanation for the bulb: apparently there is no potential difference when there is no current flow.  From Investigation I we know that when the light is off, the current is zero.  Apparently the voltage is zero also when the light is off.

Figure L4-3: Voltage vs. Time and Current vs. Time graph for Battery-bulb circuit of Figure 4-13 for Activity 2-4.

Question 2-5:  The current is zero when the switch is open and the current is non-zero when the switch is closed (and the bulb is lit). The voltage across the battery is constant (does not change when the switch is opened or closed).

Question 2-6:  Yes, the current changed significantly. (The change should be 15-20%.)

Question 2-7: The voltage across the battery did not change significantly.

Question 2-8: The battery appears to be a source of constant voltage, not current.

Investigation 3:  An Analogy to Current Flow and Resistance

Equipment and Supplies

mechanical model for current flow in a circuit (optional)

RealTime Physics Electricity and Magnetism experiment configuration files

Overview

In Investigation 3, students are introduced to a microscopic model for current flow in a circuit.  They are then led to develop analogical models for batteries, current, electric charge and electric energy in a circuit based on a two-dimensional mechanical analog.

Extension 3-1: Drawing an Analogy

Question E3-1:  The positive end of the battery provides charge carriers to the circuit at a high potential.  As they progress through the circuit the potential of the charge carriers drop until they reach the lowest potential, at the negative end of the battery. The person lifting the bowling balls represents the battery action. The balls represent the electric charges, and their motion represents the current. The balls collide with the pegs on the way down the ramp.

Question E3-2:  The “energy” given to the bowling balls by the “battery” is given up in the collisions, and is lost as the bowling balls drop to the lower potential.  The energy loss is exhibited in the circuits as heat and as light emitted from the bulb. So, the filament of the bulb is represented by the ramp and pegs.

Question E3-3: The electric current is the number of charge carriers which pass by a point in a given time. This model shows that the number of bowling balls passing by any given point per unit time is constant everywhere in the circuit.  The bowling balls are not ‘used up,’ but their potential energy is used up.

Question E3-4:  Electrons (and the bowling balls in the model) actually do accelerate for very short periods of time, but during each collision, the velocity of the charge carriers is reduced or changed in direction. Many collisions take place in a very short time. The average velocity of the carriers is a constant because they are continually accelerating and then colliding.

Question E3-5: The ball current would double. The drift velocity could be increased by increasing the slope of the ramp. The drift velocity could also be increased by reducing the number of pegs on the ramp, thus reducing the number of collisions that the ball makes per unit time.

Homework for Lab 4:  Batteries, Bulbs and Current

1.	Is there any difference between the static charges generated by rubbing rods with fur or silk and the charges that flow (from a battery) through wires in an electric circuit?  Give evidence for your answer.

No.  The discharging and charging of the electroscope were similar for the different approaches; whether the electroscope was charged with a battery or by a rod which had been charged by rubbing, the electroscope behaved the same.

a.	The current flows from the battery, through wire A, through the bulb, and then back to the battery through wire B.

True.  We measured current through wire A and through wire B and saw that it was equal when the light bulb was lighted.  Thus current is not ‘used up’ as it flows through a circuit.

b.	Since current is used up by the bulb, the current in wire B is smaller than the current in wire A.

False.  The observations mentioned in A above show that current is not used up as it flows through a circuit.  The magnitude of the current through wire A is the same as the current through wire B.

c.	The current flows toward the bulb in both wires A and B.

False.  Our observations showed that the current flows toward the bulb from the positive end of the battery, and it flows away from the bulb toward the negative end of the battery.

d.	If wire B is disconnected, but wire A is left connected, the bulb will still light, but if wire A is disconnected and wire B is left connected, the bulb will not light.

False.  A bulb will light only if a circuit is fully connected in a closed loop.

e.	A current sensor will read the same magnitude if connected to measure the current in wire A, and then connected to measure the current in wire B.

True.  The current through the circuit is the same at every point in the circuit.  This result was observed in Activity 1-5.

3.	Name the circuit element is represented by each of the following symbols.

A. Battery     B. Switch     C. Wire     D.  Bulb

4.	Use the symbols in Question 3 to draw at least one of the circuits you worked on in Activity 2-2. Here is the diagram for the Christmas tree bulbs.

5.	Draw below a circuit diagram for the circuit in Question 2 with one current sensor hooked up to measure the current in wire A and a voltage sensor hooked up to measure the voltage across the light bulb. Also include a switch in the circuit to turn the bulb on and off.  Use correct symbols for all circuit elements.

CS

VS

6. 	Consider the two circuits below. All bulbs and all batteries are identical. Compare

the voltage across the battery in the left circuit to that in the right circuit. Describe the evidence in this lab for your answer.

The voltage across the battery in the left circuit is the same as the voltage across the battery in the right circuit.