SECTION L8:  LAB 8—Introduction to Capacitors and RC circuits

General Introduction

In these investigations students are introduced to the concept and definition of capacitance and learn how to measure it with a digital multi-meter. Students initially investigate the parallel plate capacitor to discover how the capacitance of parallel plates is related to the area of the plates and the separation between them.  They then study combinations of capacitors wired in parallel and wired in series to develop a model for the total capacitance of the combinations. Students then study the effect of a capacitor on a circuit with a resistor and a voltage source, and discover the effect of a capacitor on transients in a circuit: that is, students discover how the charge on a capacitor, the electric current through it, and the voltage across it change with time.

Lab 8 Equipment and Supplies List

•	a collection of old capacitors to look at

• 	2 sheets of aluminum foil 20 cm x 20 cm

• 	pages in a "fat" textbook or telephone book

•	one or several bricks or other massive objects

•	digital multi-meter with a capacitance mode and clip leads

• 	ruler with a centimeter scale

• 	vernier calipers or a micrometer (optional)

•	computer-based laboratory system

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	2 different capacitors (each about 0.1 F)

•	multi-meter with a capacitance mode

•	6 alligator clip wires

•	2 current and 2 voltage sensors

•	6-V battery

•	#133 flashlight bulb and socket

•	2 capacitors with very large capacitance (about 25,000 F)

•	single-pole-double-throw switch

•	2 22 Ω resistors

Investigation 1: Capacitance, Area, and SEPARATION

Equipment and Supplies

•	a collection of old capacitors to look at

• 	2 sheets of aluminum foil 20 cm x 20 cm

• 	pages in a "fat" textbook or telephone book

•	one or several bricks or other massive objects

•	digital multi-meter with a capacitance mode and clip leads

• 	ruler with a centimeter scale

• 	vernier calipers or a micrometer (optional)

•	computer-based laboratory system

•	RealTime Physics Electricity and Magnetism experiment configuration files

Overview

Students construct their own parallel plate capacitors out of tin foil and telephone book pages.  They learn how to measure capacitance with a digital multi-meter, and investigate how spacing and plate area affect the capacitance of a parallel plate capacitor.

Activity 1-1:  Predicting the Dependence of Capacitance on Area and Separation

Question 1-1:  The plate connected to negative terminal will have an excess of negative charge; the plate connected to the positive terminal will have an excess of positive charge.

Question 1-2:  Charges exert forces on one another over a distance: like charges repel and unlike charges attract. Therefore, the charges on these plates will feel the attractive force of unlike charges.

Question 1-3:  As the area of the plates increase, the amount of charge on each plate will increase. Electric field is a function of charge per unit area, so as the area increases, more charge should be drawn to the plates for a fixed voltage, so the capacitance is increased.

Question 1-4:  For a constant electric field, E, V = Ed. Thus, decreasing the spacing d (with the capacitor disconnected from the battery so that the charge on the plates and the field so not change) decreases the potential difference between the plates. If q is unchanged and V decreases, C = q/V should increase.

Activity 1-2: Measuring How Capacitance Depends on Area or on Separation

The table below shows data for capacitance vs. spacing.  The capacitance values were measured with a digital multi-meter using capacitance mode, the separation distances were measured with a micrometer and the length and width of the aluminum foil were measured with a ruler.

Table 8-1

| Separation | Length | Width | Area | Capacitance |
| --- | --- | --- | --- | --- |
| (m) | (m) | (m) | (m2) | (nF) |
| .0074 | .142 | .142 | .020 | .12 |
| .0091 | .142 | .142 | .020 | .098 |
| .012 | .142 | .142 | .020 | .087 |
| .013 | .142 | .142 | .020 | .077 |
| .015 | .142 | .142 | .020 | .074 |

The data are shown graphed below.

| Figure L8-1: Graph of Capacitance vs. Separation for two parallel aluminum plates of area 0.02m2 for Activity 1-2. |  |
| --- | --- |

Question 1-5:  Answer depends on student procedures.

Question 1-6: The model shows that capacitance should decrease with separation and increase with area.  The measured data are consistent with the predicted variation with separation.

Question 1-7: Answer depends on student procedures.

Question 1-8:  The data are consistent with an inverse proportionality with d as indicated by the equation.

Question 1-9:  For d=0.0115m,  theory predicts

C = (8.85x10-12C2/Nm2)(.02m2 )/.0115m = .015nF.  The measured value was .087nF, which agrees to within an order of magnitude.

Question 1-10:  C = 0A/d= 0L2/d  =>  L2 = C d/0

L2 = (1F)(.001m)/8.85x10-12 = 1.13x108m2   and L=104m = 6.25 miles/side

Investigation 2: Capacitors in series and parallel

Equipment list

•	2 different capacitors (each about 0.1 F)

•	a multi-meter with a capacitance mode

•	6 alligator clip leads

Overview

Students discover the model for capacitance value when capacitors are combined in series and parallel.  Students use both direct physical reasoning and measurements to develop the model.

Activity 2-1: Equivalent Capacitance for Parallel Connection of Capacitors

1.	Measured capacitance of each capacitor with the multi-meter:

C1:   48.4nF	C2:    41.5nF

2.	Measured equivalent capacitance of the parallel combination of the two capacitors.	Ceq:   89.8nF

Question 2-1:  For parallel combinations the relationship appears to be: Ceq=C1+C2.

Question 2-2:  The agreement was very close.

3.	Measured equivalent capacitance of the series combination of the two capacitors:	Ceq:   22.3nF

Question 2-3:  For the measured capacitance values:

1/Ceq= 1/48.4+1/41.5 = 0.0207 + 0.0241 = 0.0448; Ceq= 22.3 nF--in excellent agreement with the measured value.

Question 2-4:  Answer depends on student prediction.

Investigation 3: Charge buildup and decay in capacitors

Equipment List

•	computer-based laboratory system

•	2 current and 2 voltage sensors

•	RealTime Physics Electricity and Magnetism experiment configuration files

•	6-V battery

•	#133 flashlight bulb and socket

•	2 capacitors with very large capacitance (about 25,000 F)

•	6 alligator clip wires

•	single-pole-double-throw switch

•	2 22 Ω resistors

Overview

Students observe behavior of circuits with batteries, bulbs and capacitors.  The exponential decay of current through the circuit and voltage across the capacitor after current transitions is observed first qualitatively in light bulbs and then quantitatively with voltage and current sensors.

Activity 3-1: Observations with a Capacitor, Battery, and Bulb

Question 3-1:

Position 1						Position 2

Question 3-2: Students may draw a linear or a curved decaying line. The main point is the decay of brightness over time.

Question 3-3:  Students may draw a linear or a curved decaying line. The main point is the decay of brightness over time.

Question 3-4:  Current flows through the bulb in both cases. In the first case the current flows until the capacitor is charged to the same potential as the battery. At that point there is no potential drop across the bulb, so current does not flow through it. In the second case, there is no battery. When the switch is switched to 2, current flows through the circuit until the capacitor is discharged, leaving no potential difference across the bulb. The current flows in different directions in each case.

| Figure L8-2: Graph of Current vs. Time and Voltage vs. Time for capacitor-bulb circuit for Activity 3-1. |  |
| --- | --- |

Question 3-5:  Student answers will vary, depending on their predictions.

Question 3-6: When the switch is switched to position 2 a potential difference exists across the bulb. Current flows due to this potential difference and therefore the bulb lights. As time goes on the potential difference decreases. As seen earlier in the lab, as the potential difference across the bulb decreases so does the current.  In this case the reduced potential difference leads to reduced current that leads to reduced brightness of the bulb.

Activity 3-2: The Rise and Decay of Voltage in an RC Circuit.

| Figure L8-3: Graph of Current vs. Time and Voltage vs. Time for capacitor-resistor circuit for Activity 3-2. |  |
| --- | --- |

4.	Determine of the time for the potential difference on the capacitor to decay to 37% of its initial value (when switch is moved from position 1 back to 2).

Initial value:  	    5.52V	Time:  8.04 sec

37% of initial value:  2.04V	Time:  8.74 sec

Time constant:  8.74-8.04 = 0.7 sec

Question 3-7:  For the bulb, the current vs. time plot will look less like an exponential decay at the top of the graph. This is due again to the non-linearity of the bulb resistance. As the bulb filament gets hotter, the resistance of the filament increases. The curve for the resistor is definitely an exponential decay.

Question 3-8:  The curves should look very much like an exponential decay.

Question 3-9: V = Vo e- t/RC= 0.37Vo

e- t/RC= 0.37 or - t/RC = ln(0.37)= -1.0

t  = -RC  = (22)(32700x10-6) = 0.72 sec—in excellent agreement!

Extension 3-3: Does the Observed Decay Curve Fit Theory?

Question 3-10: The curves should fit the exponential well.  Care must be taken to select data points only AFTER the decay begins.

Question 3-11:  RC measured from graph = 0.7sec.   Calculated RC:

(22 (32700F) = 0.72 sec—an excellent agreement!

Extension 3-4: Decay with a Larger Resistance and Capacitance

Measurement of time coonstant for circuit with increased resistance (R=44):

Initial value:    	5.45V		Time:  9.76 sec

37% of initial value:   2.01V	Time:   11.15 sec

Time constant:   11.15 sec - 9.76 sec = 1.4 sec

Question E3-12: The time constant increases with increased resistance (RC increases).

The data for the circuit with an increased capacitance and the increased resistance (44) are given below.

Initial value:   5.32 V		Time:   11.32 sec

37% of initial value:   1.97 V	Time:   14.10 sec

Decay time:   14.10 sec - 11.32 sec = 2.8 sec

Question E3-13:  The time constant increases (RC increases).

HOMEWORK FOR LAB 8:  Introduction to Capacitors and RC Circuits

1.	Explain in terms of the charge, electric field and potential difference how the capacitance of a parallel plate capacitor depends on the area and separation of the plates in the equation C =  eoA/d.

If the area of the plates is increased without changing the amount of charge (plates disconnected after charging by battery), then the charge per unit area is decreased, the electric field between the plates is decreased, and so V is decreased. Capacitance is inversely proportional to V (C  =  q/V),  so the capacitance is increased.

If d is increased with the amount of charge and the area remaining constant then electric field E is unchanged, V (=Ed) is increased, and C is decreased.

(For a parallel plate capacitor, E is proportional to q/A, so increasing the separation between the plates of a capacitor (as long as separation doesn’t approach plate size) doesn't change the electric field.)

2.	If a 1.5-V battery is connected to a 250-mF capacitor, how much excess charge is there on the capacitor plates?

C  =  q/V --->  q  =  CV  =  250 F x 1.5 V  = 375 x 10-6 C  =  375 C.

a.	Both capacitors have the same amount of charge on their plates.

True:  Since they are in series - whatever charge flows onto one must be matched by an equal amount flowing away from the other.

b.	The voltages across both capacitors are the same.

False:  Since V = Q/C:  V1  =  Q/C1 does not equal V2  =  Q/C2.  (The capacitors have different capacitance).

c.	The sum of the voltages on the two capacitors equals the voltage of the battery.

True:  This is a series circuit and we have seen with bulbs and resistors that the sum of the potential drops around the loop must equal the potential rise in the battery.  Hence: V = V1 + V2.

4.	Find the equivalent capacitance of each of the following combinations of capacitors.  (All capacitances are in mF.)

a.

b.	How would your observations be changed if the capacitor were twice as large?

The bulb would glow twice as long.

c.	How would your observations be changed if the bulb had half as much resistance?

The bulb would glow more brightly (more current) but only for half as long as in the initial case.

6.	Sketch a graph of the current as a function of time in the circuit in Question 5 after the switch is closed.  Also sketch a graph of the voltage across the capacitor as a function of time.

=RC=(0.023)(47)=1.1 s