SECTION L5:  LAB 5—polarized light

General Introduction

This first physical optics lab deals qualitatively and quantitatively with polarized light.

Lab 5 Equipment and Supplies List

- two Polaroid filter disks with angle markings on them

- light source (e.g., flashlight)

- computer-based laboratory system

- RealTime Physics Light and Optics experiment configuration files

- light sensor

- rotary motion sensor

- stand

- large Polaroid disk

- small lens and holder (optional)

- protractor

- glass plate and holder

Two small Polaroid filter disks with angle markings on them:  The Pasco Scientific (www.pasco.com) Polarizer Set, # OS-8482, shown in Figure L5-1, is perfect for use in these lab activities. Any similar Polaroid disks will work fine. The angular markings are nice, but not essential. These Polaroid Disks are part of the Introductory Optics System from Pasco, # OS-8500, which also includes a small lens and holder, the cylindrical lens, green filter, lenses, arrow-shaped light source, optical rail, holders and screen used in Lab 3, the mirror set for Lab 4, and the Diffraction Plate for Lab 6.

Figure L5-1: Pasco Polaroid Disks.

Light Sensor: Light sensors have been discussed in Section L1.

Rotary motion sensor: For the Vernier (www.vernier.com) LabPro interface, use the Vernier Rotary Motion Sensor, #RMS-BTD, pictured in Figure L5-2. For Pasco Science Workshop interfaces, use the #CI-6538 Rotary Motion Sensor. For the Pasco Passport system, use the #PS-2120 Rotary Motion Sensor.

Figure L5-2: Vernier (or Pasco) Rotary Motion Sensor

Large Polaroid disk: This disk will be mounted on the Rotary Motion Sensor. See Figures L5-3 and L5-4. It should be about 15 cm in diameter with a small hole at the center to mount on the sensor’s shaft. It can be fashioned of any Polaroid material, available for example from Edmund Scientific (scientificsonline.com), #3038491 or #3060638.

Investigation 1: polarizing light with a polarizing filter

Equipment and Supplies

- two Polaroid filter disks with angle markings on them

- light source (e.g., flashlight)

- computer-based laboratory system

- RealTime Physics Light and Optics experiment configuration files

- light sensor

- rotary motion sensor

- stand

- large Polaroid disk

- small lens and holder (optional)

Overview

After some qualitative observations on polarized light, the light sensor and rotary motion sensor are used as an analyzer for polarized light, and Malus’ law is examined quantitatively.

Activity 1-1: Looking at Light through a Polaroid Filter

Question 1-1: The Polaroid filter reduced the intensity of the light from the flashlight somewhat.

Question 1-2: There was no additional change as the Polaroid was rotated.

Figure L5-5 shows how the small Polaroid is mounted on the flashlight.

Question 1-3: Now when the other Polaroid is rotated, the intensity of light reaching the eye is almost as large as before for some angles and zero for other angles. As the Polaroid is rotated, the intensity varies between these two extremes.

Question 1-4: The 0° marker is also pointing upward or downward when the transmitted intensity is maximum.

Question 1-5: When the Polaroid is rotated at right angles, the transmitted intensity drops to zero.

Question 1-6: In between, the intensity is somewhere in between its maximum value and zero.

Activity 2-1: Explaining Your Observations of Polarization

Question 1-7: There must be an equal number of electric field vectors pointing in all directions perpendicular to the direction of propagation of the light waves. The direction must be random.

Question 1-8: Only light with components of electric field pointing parallel to the axis of the Polaroid is transmitted through the Polaroid. Therefore all light that gets through the Polaroid has its electric field aligned parallel to the axis of the Polaroid.

Question 1-9: When the axes are parallel, the light transmitted by the first Polaroid has all of its electric field vectors parallel to the axis of the second Polaroid. Therefore the light is transmitted through the second Polaroid. When the axis of the second Polaroid is perpendicular to the axis of the first Polaroid, all of the electric field vectors of the light transmitted by the first Polaroid are perpendicular to the axis of the second Polaroid, so nothing gets through.

Question 1-10: When the electric field vector points at an angle somewhere between 0° and 90° to the axis of the second Polaroid, only the vector component of the electric field parallel to the axis gets through. Therefore, the intensity is less than when the axes are parallel, but is not zero.

Activity 1-3: Dependence of Transmitted Intensity on Angle

Figure L5-3 shows the experimental setup with the light sensor, rotary motion sensor and flashlight. Figure L5-4 is a close-up of the sensors, while Figure L5-5 shows the Polaroid mounted on the flashlight.

Figure L5-3: Setup for observation of polarized light (Activity 1-3).

Figure L5-4: Light sensor with Polaroid mounted on rotary motion sensor.

Figure L5-6 shows the results rotating the large Polaroid through 360°.

Question 1-11: Answer depends on student prediction.

Activity 1-4: Mathematical Model for Transmitted Intensity as a Function of Angle

Since the graph looks like a sine function, students will jump to that conclusion. If it is modeled carefully, it is clear that the function is the square of a sinusoidal function. The model for the calculated intensity in Figure L5-7 is                              I = 800*cos(“Position” + 3.14/4)^2 + 400.

Figure L5-5: Polaroid mounted on flashlight to produce a source of polarized light.

Figure L5-6: Intensity as a function of angle (Activity 1-3).

Question 1-12: It appears from the model used to calculate the transmitted intensity as a function of angle that the intensity varies as cos2 .

Question 1-13: Answer depends on student prediction.

Question 1-14: The match is very good, as can be seen in Figure L5-7. The two graphs almost overlap.

Question 1-15: If the electric field makes an angle  with the axis of the Polaroid, then the component of the electric field parallel to the axis of the Polaroid is     Ecos . Since intensity depends on the square of the amplitude of a wave, the intensity transmitted by the Polaroid should vary as cos2 .

Figure L5-7: Measured and calculated intensity vs. angle (Activities 1-3 and 1-4). The mathematical model for  the calculated intensity is displayed. It is I = 800*cos(“Position” + 3.14/4)^2 + 400.

Investigation 2: other ways of polarizing light

Equipment and Supplies

- two Polaroid filter disks with angle markings on them

- light source (e.g., flashlight)

- computer-based laboratory system

- RealTime Physics Light and Optics experiment configuration files

- light sensor

- rotary motion sensor

- stand

- large Polaroid disk

- small lens and holder (optional)

- protractor

- glass plate and holder

Overview

This investigation uses the same apparatus as in Investigation 1 to explore polarization by reflection.

Activity 2-1: Analyzing Reflected Light

Since the percentage of light reflected by the glass plate is small, using a lens to focus the light onto the light sensor will help to collect more of the light. Some attention to alignment now will save time later.

The graph for this Activity should be similar to Figure L5-6.

Question 2-1: The intensity variation is very similar to that in Investigation 1 for polarized light.

Question 2-2: The un-polarized light appears to become polarized when it is reflected.

Question 2-3: Polaroid sunglasses should be especially effective when viewing reflected light, e.g., from a car windshield or surface of a lake.

Extension 2-2: Direction of Polarization of Reflected Light

Question E2-4: The polarization seems to be up and down, perpendicular to the plane of incidence of the light on the mirror. This is clear from the observations because no light gets through the small Polaroid when its axis is horizontal, and maximum gets through when its axis is vertical.

Question E2-5: The polarization seems to be perpendicular to the plane of incidence of the light on the mirror.

Extension 2-3: Brewster’s Angle

Question E2-6: There was some variation. It appears that the degree of polarization depends on the angle of incidence.

Question E2-7: Calculation depends on the glass used. Assuming n = 1.5,         tan B = 1.5, B = 56°.

Question E2-8: Answer depends on student measurements. This is a difficult measurement to do. The uncertainly in measuring the angle is relatively large, but the two angles should agree within this uncertainty.

answers to Homework for LAB 5: polarized light

- What is polarized light? How would you determine if light from some source is polarized?

Linearly polarized light is light that always has its electric field vector along the same direction. If you shine light through a Polaroid filter, and the intensity changes as you rotate the filter, then the light is polarized.

- Describe two ways of polarizing light that you observed in this laboratory.

Un-polarized light can be polarized by 1) passing it through a polarizing filter and 2) reflecting it off a glass plate (especially at Brewster’s angle).

- You view light from a flashlight through a Polaroid. Describe any intensity variation as the Polaroid is rotated. Explain your answer.

# The light from a flashlight is un-polarized, so there should be no variation as the Polaroid is rotated.

Describe any intensity variation as the flashlight is rotated.

There should also be no intensity variation as the flashlight is rotated.

- Unpolarized light travels through two pieces of Polaroid as shown below.

When the two Polaroids are rotated to 0 as shown, the transmitted intensity is I0. What percentage of I0 will be transmitted when the analyzer is rotated to 90? Explain.

No light will be transmitted. After passing through the polarizer, all the electric field vectors point up/down. When the analyzer is rotated to 90°, all the electric field vectors are perpendicular to its axis.

What percentage of I0 will the transmitted intensity be if the analyzer is rotated to 30? Show your calculation.

I = Iocos2 = Io cos2 (30°) = 0.75 Io. 75% will be transmitted.

- Light is reflected off the rear windshield of a car.

When the Polaroid is rotated so that the 0 or 360 mark is at the top, the transmitted intensity is minimum, while at 90 and 270 it is maximum. Explain why this variation in intensity occurs.

As the light is reflected from the windshield it is (partially) polarized. Therefore as the Polaroid is rotated, the transmitted intensity varies.

6.	What is the direction of the axis of polarization of the light reflected from the windshield in (5)—up and down or side-to-side?

The reflected light is polarized with its electric field vector perpendicular to the plane of incidence. So the light has its electric field vectors pointing horizontal. These will only be completely transmitted if the axis of the Polaroid is also horizontal, so the angle at the top should be 90° or 270°.

If you were designing Polaroid sunglasses, in which of the following directions should the axis of the Polaroid point? Explain your answer.

The answer is A. Since the reflected light has its electric field vectors pointing horizontally, the glasses in A will not transmit any of the reflected light.

- Lucite has index of refraction 1.49. What is Brewster’s angle for Lucite? Show your calculation.

tan B = 1.49, B = 56°.

If unpolarized light is incident on the surface of a piece of Lucite at this angle, describe the degree of polarization of the reflected light.

The reflected light will be completely polarized.