SECTION CS:  General Computer Hardware and Software Notes

Notes on Microcomputer-Based Laboratory (MBL) Systems

A number of different systems are available for microcomputer supported collection, display and analysis of data.  For example there are the LabPro and LabQuest interfaces from Vernier Software,1 and the Science Workshop interface series from PASCO Scientific.2  Any of these will work with both Macintosh and Windows computers.

Light and rotary motion sensors required for the RealTime Physics Light and Optics  experiments are available from both of these suppliers for both of these interfaces.

Software packages are available with each of these systems.  Vernier’s  Logger Pro (for Macintosh and Windows) and PASCO’s Data Studio software both allow the use of a motion detector and a force probe, or two force probes.

Since RealTime Physics Light and Optics is designed to be independent of the MBL hardware/software system, specific software directions are not included in the curriculum. However, since the students are using the same user-friendly software and tools each week, it is our experience that they quickly master its use.

In addition, Experiment Configuration Files allow students to set up the software by just opening one file with appropriate settings for each activity. RealTime Physics Light and Optics Experiment Configuration Files for Logger Pro are on the disk from Vernier Software and Technology, in the Experiments folder. For Data Studio, the Experiment Configuration Files can be downloaded from the PASCO website, free of charge. All of the setup features are given in Appendix A of this Guide.

Which MBL system should you adopt? Both Vernier and PASCO interfaces and software are compatible with most of the activities in RealTime Physics Light and Optics. The one exception is Module 1, Lab 10, and Module 3, Labs 1 and 10, since only Vernier LoggerPro has video analysis incorporated. (There are also some exceptions for other RTP modules. For example, since PASCO does not have a heat pulser available, many of the activities in Module 2: Heat and Thermodynamics, require special considerations with PASCO.)

In selecting the interface, software and hardware system several considerations are important:

1.	Do the interface, software and hardware have the flexibility you will need for your present and future laboratory needs? Is the system expandable to other areas of physics that you wish to include in the laboratory?

2.	Is the software user friendly?  Does it support your pedagogical goals?  Are data easily displayed in a manner that supports collaborative, active learning in the laboratory?

3.	Are active learning curricula that meet your laboratory objectives supported by the system?

Only you can decide. We suggest that you spend enough time trying both systems with a number of activities from each module, before making a decision.

Notes on Video Analysis

Video analysis is a powerful and educationally effective tool for both advanced physics majors and introductory physics students. Students find video capture and analysis to be compelling and effective aids to learning. Vernier LoggerPro incorporates the ability to mark the positions of objects in a video, frame-by-frame, and to extract and analyze the data. This can be done with already produced movies of physical situations, or with movies that students produce in and outside of the laboratory. Video analysis is incorporated in RTP Mechanics Lab 1: Two-Dimensional Motion, and RTP Electricity and Magnetism Lab 1: Electric Charges, Forces and Fields and Lab 10: Electromagnetism.  If you are using PASCO MBL software, you will need to also have video analysis software. Purchasing one copy of LoggerPro is a reasonable approach, since Vernier’s generous site license allows one copy to be used in all classes on your campus, and even allows students to take a copy for use at home.

References

phone plug.v2 001

1 Vernier Software and Technology, 13979 SW Millikan Way, Portland, OR 97225-2886, 503-277-2299 (888-837-6437), http://www.vernier.com.

2 PASCO Scientific, PO Box 619011, 10101 Foothills Blvd., Roseville, CA 95747-7100, 916-786-3800 (800-772-8700), http://www.pasco.com.